# Device Io Control functions for Sound Devices

Note : Normal errors associated with the NtDeviceIoControlFile call (eg unable to lock data, insufficient memory etc) are not discussed here.

## Device Types

The following device types are used when calling IoCreateDevice :

FILE_DEVICE_WAVE_IN - wave input devices

FILE_DEVICE_WAVE_OUT - wave output devices

FILE_DEVICE_MIDI_IN - Midi input devices

FILE_DEVICE_MIDI_OUT - Midi output devices

## Access rights for sound devices

Write access gives the ability to send data to the device.  Generally the driver may only allow one write handle or perhaps up to the number of logical devices supported by the device.

Read access must in general ALWAYS be possible so that windows style queries (xxxx_GET_CAPABILITIES) can be executed.

As a special dispensation the xxx_SET_VOLUME functions are allowed with Read access provided any writers have granted Write share access.  This is to allow writing of mixer applications without undue complexity.

## Header files

Sound IOCTLs are currently defined in NTDDWAVE.H (for wave devices) and NTDDMIDI.H (for midi devices).  These files also define the default device names for this kind of device :

DD_WAVE_IN_DEVICE_NAME

DD_WAVE_OUT_DEVICE_NAME

DD_MIDI_IN_DEVICE_NAME

DD_MIDI_OUT_DEVICE_NAME

The Kernel device name naming convention is followed whereby a device of a given type can be found by appending '0', '1', ... to the generic name.

Definitions of wave headers and device capabilities structures are in WINMM.H.

## Background on Ioctls

## Ioctl types :  The bottom 2 bits determine the type  -

00 - IO subsystem copies the input data into internal storage.  Output size is determined by IOCB.Information set by driver.

01 - Input operation (input to driver - presumably output to device).  Input treated as in case 0, output is probed and locked (driver's responsibility to map).

02 - as 01 but input and output reversed.

03 - IO subsystem does nothing with input and output buffers.

Note that IOCB.Information is an output.  It usually contains the number of bytes returned or 0 if an error occurred.  This is actually required if the IOCTL type (least significant 2 bits) is 0.

IOCB.IoStatus may also contain STATUS_CANCELLED for any asynchronous IOCTL.

## General return values

Some IOCTLs can return STATUS_BUFFER_TOO_SMALL.  In this case no data is returned, the Information field in the IOCB contains 0.

## Wave IOCTLs

| IOCTL_WAVE_QUERY_FORMAT IOCTL_WAVE_SET_FORMAT |  | Input is PCMWAVEFORMAT structure. Return status determines if format supported. If the control code is IOCTL_WAVE_SET_FORMAT and the specified format is supported then subsequent data in IOCTL_WAVE_PLAY data is played in the specified format. After the wave output device is opened if no IOCTL_WAVE_SET_FORMAT is received by the device then wave output is played at a device specified default rate. NB Operation is SYNCHRONOUS but if wave data is being played then the time the new format takes effect is UNDEFINED. On the soundblaster the default rate is 11.025Khz and new formats take effect whenever output is restarted either due to a reset or due to completion of all input data. |  |  |
| --- | --- | --- | --- | --- |
|  | Input buffer size |  | sizeof(PCMWAVEFORMAT) |  |
|  | Input buffer |  | PCMWAVEFORMAT structure to test |  |
|  | Output buffer size |  | 0 or sizeof(ULONG) |  |
|  | Output buffer |  | Nothing. |  |
|  | IOCB.Information |  | 0 |  |
|  | IOCB.Status |  | STATUS_SUCCESS | Wave format is supported |
|  |  |  | STATUS_NOT_SUPPORTED | Wave format not supported (includes input buffer size not sizeof(PCMWAVEFORMAT) since other formats may not have this size). |

| IOCTL_WAVE_GET_CAPABILITIES |  | The output buffer is filled with WAVEOUTCAPS data for the wave device. |  |  |
| --- | --- | --- | --- | --- |
|  | Input buffer size |  | 0 |  |
|  | Input buffer |  | None |  |
|  | Output buffer size |  | any |  |
|  | Output buffer |  | WAVEOUTCAPS data for wave output devices, WAVEINCAPS for wave input devices. |  |
|  | IOCB.Information |  | Amount of data returned |  |
|  | IOCB.Status |  | STATUS_SUCCESS | Data returned OK |

| IOCTL_WAVE_GET_STATE |  | The output buffer is filled with a ULONG representing the current wave output state : WAVE_DD_IDLE Nothing playing WAVE_DD_PLAYING Playing wave data WAVE_DD_RECORDING Recording wave data WAVE_DD_STOPPED Operation suspended (by user) |  |  |
| --- | --- | --- | --- | --- |
|  | Input buffer size |  | 0 |  |
|  | Input buffer |  | None |  |
|  | Output buffer size |  | sizeof(ULONG) |  |
|  | Output buffer |  | Current state |  |
|  | IOCB.Information |  | sizeof(ULONG) |  |
|  | IOCB.Status |  | STATUS_SUCCESS | Data returned OK |

| IOCTL_WAVE_SET_STATE |  | The input buffer contains the new state change value : WAVE_DD_STOP Temporarily suspend playing WAVE_DD_PLAY Continue playing WAVE_DD_RECORD Start recording WAVE_DD_RESET Cancel all input or output. Note that for wave output devices WAVE_DD_RESET sets the state to WAVE_DD_IDLE (ie it cancels a WAVE_DD_STOPPED state, as does WAVE_DD_RESUME). WAVE_DD_STOP and WAVE_DD_RESET can be asynchronous. The effect of supplying more wave data or requesting other state changes before they complete is safe but undefined (eg the data may be ignored, used etc). For wave input devices the oldest IOCTL_WAVE_RECORD is completed (if there is one), even if there is no data to return. This is for compatibility with older systems. New IOCTL_WAVE_PLAY data does not cancel a WAVE_DD_STOPPED state. |  |  |
| --- | --- | --- | --- | --- |
|  | Input buffer size |  | sizeof(ULONG) |  |
|  | Input buffer |  | New state |  |
|  | Output buffer size |  | 0 |  |
|  | Output buffer |  | None |  |
|  | IOCB.Information |  | 0 |  |
|  | IOCB.Status |  | STATUS_SUCCESS | State changed OK |
|  |  |  | STATUS_PENDING | Operation not yet complete. |
|  |  |  | STATUS_DEVICE_BUSY | Another state change is incomplete. |

| IOCTL_WAVE_GET_POSITION |  | The output buffer is filled a WAVE_DD_POSITIOn structure representing the position into the sound being played both in terms of bytes and samples. |  |  |
| --- | --- | --- | --- | --- |
|  | Input buffer size |  | 0 |  |
|  | Input buffer |  | None |  |
|  | Output buffer size |  | >= sizeof(WAVE_DD_POSITION) |  |
|  | Output buffer |  | Number of sound samples and number of bytes representing those samples played since start of playing or last RESET (not including STOPPED time). The position is given as accurately as the driver can reasonably manage. |  |
|  | IOCB.Information |  | sizeof(WAVE_DD_POSITION) |  |
|  | IOCB.Status |  | STATUS_SUCCESS | Data returned OK |
|  |  |  | STATUS_BUFFER_TOO_SMALL | Output buffer smaller than sizeof(LARGE_INTEGER). |

| IOCTL_WAVE_PLAY |  | The output buffer is added to the list of buffers to be played. Only valid on wave output devices. If there is no format set since the device was Opened then a default format is assumed. This call IOCTL is usually asynchronous. Provided the input is not synchronously rejected IOCTL_WAVE_PLAY IOCTLs are completed in the order of submission. |  |  |
| --- | --- | --- | --- | --- |
|  | Input buffer size |  | 0 or >= sizeof(WAVE_DD_HEADER) |  |
|  | Input buffer |  | Either none or a WAVE_DD_HEADER structure with loop control information. |  |
|  | Output buffer size |  | any |  |
|  | Output buffer |  | Wave data to play. |  |
|  | IOCB.Information |  | Amount of data played. This is also valid if the IoStatus is STATUS_CANCELLED. |  |
|  | IOCB.Status |  | STATUS_SUCCESS | Data played. |
|  |  |  | STATUS_BUFFER_TOO_SMALL | Input buffer not large enough (and not 0 size). |

| IOCTL_WAVE_SET_VOLUME |  | Sets the volume for subsequent playing. |  |  |
| --- | --- | --- | --- | --- |
|  | Access Rights |  | FILE_DATA_READ if file not opened for write without FILE_SHARE_WRITE specified. |  |
|  | Input buffer size |  | >=sizeof(WAVE_DD_VOLUME) |  |
|  | Input buffer |  | WAVE_DD_VOLUME for the volume |  |
|  | Output buffer size |  | 0 |  |
|  | Output buffer |  | None |  |
|  | IOCB.Information |  | 0 |  |
|  | IOCB.Status |  | STATUS_SUCCESS | Volume set |
|  |  |  | STATUS_BUFFER_TOO_SMALL | Not enough input data |
|  |  |  | STATUS_NOT_SUPPORTED | Setting volume not supported |

| IOCTL_WAVE_SET_PITCH |  | Sets the pitch for subsequent playing. |  |  |
| --- | --- | --- | --- | --- |
|  | Input buffer size |  | >=sizeof(WAVE_DD_PITCH) |  |
|  | Input buffer |  | WAVE_DD_PITCH for the pitch |  |
|  | Output buffer size |  | 0 |  |
|  | Output buffer |  | None |  |
|  | IOCB.Information |  | 0 |  |
|  | IOCB.Status |  | STATUS_SUCCESS | New Pitch set |
|  |  |  | STATUS_BUFFER_TOO_SMALL | Not enough input data |
|  |  |  | STATUS_NOT_SUPPORTED | Setting pitch not supported |

| IOCTL_WAVE_SET_PLAYBACK_RATE |  | Sets the playback rate for subsequent playing. |  |  |
| --- | --- | --- | --- | --- |
|  | Input buffer size |  | sizeof(WAVE_DD_PLAYBACK_RATE) |  |
|  | Input buffer |  | WAVE_DD_PLAYBACK_RATE for the playback rate |  |
|  | Output buffer size |  | 0 |  |
|  | Output buffer |  | None |  |
|  | IOCB.Information |  | 0 |  |
|  | IOCB.Status |  | STATUS_SUCCESS | New rate set OK |
|  |  |  | STATUS_BUFFER_TOO_SMALL | Not enough input data |
|  |  |  | STATUS_NOT_SUPPORTED | Setting playback rate not supported |

| IOCTL_WAVE_GET_VOLUME |  | Gets the current volume setting for subsequent playing. If volume setting is not supported then maximum volume (0xFFFFFFFF) is returned. |  |  |
| --- | --- | --- | --- | --- |
|  | Input buffer size |  | 0 |  |
|  | Input buffer |  | None |  |
|  | Output buffer size |  | >=sizeof(WAVE_DD_VOLUME) |  |
|  | Output buffer |  | WAVE_DD_VOLUME structure |  |
|  | IOCB.Information |  | sizeof(WAVE_DD_VOLUME) |  |
|  | IOCB.Status |  | STATUS_SUCCESS | Data returned OK |
|  |  |  | STATUS_BUFFER_TOO_SMALL | Not enough room for output data |

| IOCTL_WAVE_GET_PITCH |  | Gets the current pitch |  |  |
| --- | --- | --- | --- | --- |
|  | Input buffer size |  | 0 |  |
|  | Input buffer |  | None |  |
|  | Output buffer size |  | >=sizeof(WAVE_DD_PITCH) |  |
|  | Output buffer |  | WAVE_DD_PITCH representing current pitch setting |  |
|  | IOCB.Information |  | sizeof(WAVE_DD_PITCH) |  |
|  | IOCB.Status |  | STATUS_SUCCESS | Data returned OK |
|  |  |  | STATUS_BUFFER_TOO_SMALL | Not enough room for output data |

| IOCTL_WAVE_GET_PLAYBACK_RATE |  | Returns the playback rate for subsequent playing. |  |  |
| --- | --- | --- | --- | --- |
|  | Input buffer size |  | 0 |  |
|  | Input buffer |  | None |  |
|  | Output buffer size |  | >=sizeof(WAVE_DD_PLAYBACK_RATE) |  |
|  | Output buffer |  | WAVE_DD_PLAYBACK_RATE representing current playback rate setting |  |
|  | IOCB.Information |  | sizeof(WAVE_DD_PLAYBACK_RATE) |  |
|  | IOCB.Status |  | STATUS_SUCCESS | Data returned OK |
|  |  |  | STATUS_BUFFER_TOO_SMALL | Not enough room for output data |

| IOCTL_WAVE_RECORD |  | The output buffer is added to the list of buffers to be recorded into. Only valid on wave input devices. If there is no format set since the device was opened then a default format is chosen. This call IOCTL can be either asynchronous or synchronous depending on whether there is sufficient data already recorded to fulfill the request when it is issued. Provided the input is not synchronously rejected IOCTL_WAVE_RECORD IOCTLs are completed in the order of submission. The device should buffer data so there is no need to have more than one buffer outstanding at any one time, nor any need to provide a buffer before setting the state to WAVE_DD_RECORDING. |  |  |
| --- | --- | --- | --- | --- |
|  | Input buffer size |  | 0 |  |
|  | Input buffer |  | None |  |
|  | Output buffer size |  | any |  |
|  | Output buffer |  | Wave data recorded (on completion). |  |
|  | IOCB.Information |  | Amount of data recorded. This is also valid if the IoStatus is STATUS_CANCELLED. If recording is stopped by a IOCTL_WAVE_SET_STATE with a state change of WAVE_DD_STOP or WAVE_DD_RESET or the device is closed then an incomplete buffer can be returned. |  |
|  | IOCB.Status |  | STATUS_SUCCESS | Data recorded. |
|  |  |  | STATUS_BUFFER_TOO_SMALL | Input buffer not large enough (and not 0 size). |

| IOCTL_WAVE_BREAK_LOOP |  | Cancels any currently playing loop. The loop is cancelled after its current iteration is finished. This is really a user-interface feature since it's hard to synchronize accurately to tell whether the loop is actually playing, which iteration it's on etc. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Input buffer size |  | 0 |  |  |  |
|  | Input buffer |  | None |  |  |  |
|  | Output buffer size |  | 0 |  |  |  |
|  | Output buffer |  | None |  |  |  |
|  | IOCB.Information |  | 0 |  |  |  |
|  | IOCB.Status |  | STATUS_SUCCESS | Any loop running will stop |  |  |

## MIDI Ioctls

| IOCTL_MIDI_GET_CAPABILITIES |  | The output buffer is filled with MIDIOUTCAPS data for the midi device. |  |  |
| --- | --- | --- | --- | --- |
|  | Input buffer size |  | 0 |  |
|  | Input buffer |  | None |  |
|  | Output buffer size |  | any |  |
|  | Output buffer |  | MIDIOUTCAPS data for Midi output devices, MIDIINCAPS for Midi input devices. |  |
|  | IOCB.Information |  | Amount of data returned |  |
|  | IOCB.Status |  | STATUS_SUCCESS | Data returned OK |

| IOCTL_MIDI_GET_STATE |  | The output buffer is filled with a ULONG representing the current MIDI output state : MIDI_DD_IDLE Nothing playing MIDI_DD_PLAYING Playing MIDI data MIDI_DD_RECORDING Recording MIDI data MIDI_DD_STOPPED Operation suspended (by user) Some devices may decide to implement midi output synchronously in which case the state returned is always MIDI_DD_IDLE. |  |  |
| --- | --- | --- | --- | --- |
|  | Input buffer size |  | 0 |  |
|  | Input buffer |  | None |  |
|  | Output buffer size |  | sizeof(ULONG) |  |
|  | Output buffer |  | Current state |  |
|  | IOCB.Information |  | sizeof(ULONG) |  |
|  | IOCB.Status |  | STATUS_SUCCESS | Data returned OK |

| IOCTL_MIDI_SET_STATE |  | The input buffer contains the new state change value : MIDI_DD_STOP Temporarily suspend playing or recording MIDI_DD_PLAY Continue playing MIDI_DD_RECORD Start recording MIDI_DD_RESET Cancel all input. MIDI_DD_RESET cancels any outstanding requests. For Midi output all notes are turned off. Only MIDI_DD_RESET is valid for midi output devices. For Midi input devices MIDI_DD_RECORD is required before any recording takes place. A recording device can do some (device-dependent) buffering which is important since data can come in in large numbers of short bursts so it is not practical to keep the device supplied permanently with large numbers of outstanding IOCTL_MIDI_RECORD requests. |  |  |
| --- | --- | --- | --- | --- |
|  | Input buffer size |  | sizeof(ULONG) |  |
|  | Input buffer |  | New state |  |
|  | Output buffer size |  | 0 |  |
|  | Output buffer |  | None |  |
|  | IOCB.Information |  | 0 |  |
|  | IOCB.Status |  | STATUS_SUCCESS | Data returned OK |
|  |  |  | STATUS_PENDING | Operation not yet complete. |
|  |  |  | STATUS_DEVICE_BUSY | Another state change is incomplete. |

| IOCTL_MIDI_PLAY |  | Play midi data |  |  |
| --- | --- | --- | --- | --- |
|  | Input buffer size |  | Midi data |  |
|  | Input buffer |  | Number of bytes to play |  |
|  | Output buffer size |  | 0 |  |
|  | Output buffer |  | Nothing |  |
|  | IOCB.Information |  | 0 |  |
|  | IOCB.Status |  | STATUS_SUCCESS | Data played OK |
|  |  |  | STATUS_DEVICE_BUSY | Data still playing |

| IOCTL_MIDI_RECORD |  | Record midi data. |  |  |
| --- | --- | --- | --- | --- |
|  | Input buffer size |  | 0 |  |
|  | Input buffer |  |  |  |
|  | Output buffer size |  | >= sizeof(MIDI_DD_INPUT_DATA) |  |
|  | Output buffer |  | MIDI_DD_INPUT_DATA structure |  |
|  | IOCB.Information |  | 0 |  |
|  | IOCB.Status |  | STATUS_SUCCESS | Data played OK |
|  |  |  | STATUS_BUFFER_TOO_SMALL | Buffer not big enough |

| IOCTL_MIDI_GET_VOLUME |  | Gets the current volume setting for subsequent playing. If volume setting is not supported then maximum volume (0xFFFFFFFF) is returned. |  |  |
| --- | --- | --- | --- | --- |
|  | Input buffer size |  | 0 |  |
|  | Input buffer |  | None |  |
|  | Output buffer size |  | >=sizeof(UFIXED) |  |
|  | Output buffer |  | UFIXED volume |  |
|  | IOCB.Information |  | sizeof(UFIXED) |  |
|  | IOCB.Status |  | STATUS_SUCCESS | Data returned OK |
|  |  |  | STATUS_BUFFER_TOO_SMALL | Not enough room for output data |

| IOCTL_MIDI_SET_VOLUME |  | Sets the volume for subsequent playing. |  |  |
| --- | --- | --- | --- | --- |
|  | Access Rights |  | FILE_DATA_READ if file not opened for write without FILE_SHARE_WRITE specified. |  |
|  | Input buffer size |  | >=sizeof(UFIXED) |  |
|  | Input buffer |  | UFIXED for the volume |  |
|  | Output buffer size |  | 0 |  |
|  | Output buffer |  | None |  |
|  | IOCB.Information |  | 0 |  |
|  | IOCB.Status |  | STATUS_SUCCESS | Volume set |
|  |  |  | STATUS_BUFFER_TOO_SMALL | Not enough input data |
|  |  |  | STATUS_NOT_SUPPORTED | Setting volume not supported |

| IOCTL_MIDI_CACHE_PATCHES |  | Request an internal synthesizer preload a specified set of patches. Some synthesizers are not capable of keeping all patches loaded simultaneously and must load data from disk when they receive MIDI program change messages. Caching patches ensures specified patches are immediately available. The bank structure element specifies which bank of patches should be used. Specify 0 for the default patch bank. Each element of the patch array represents one of the 128 patches and has bits set for each of the 16 MIDI channels that use that particular patch. The least significant bit represents physical channel 15. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Input buffer size |  | >=sizeof(MIDI_DD_CACHE_PATCHES) |  |  |  |
|  | Input buffer |  | MIDI_DD_CACHE_PATCHES structure |  |  |  |
|  | Output buffer size |  | 0 |  |  |  |
|  | Output buffer |  | None |  |  |  |
|  | IOCB.Information |  | 0 |  |  |  |
|  | IOCB.Status |  | STATUS_SUCCESS | Patches cached |  |  |
|  |  |  | STATUS_BUFFER_TOO_SMALL | Not enough input data |  |  |
|  |  |  | STATUS_NOT_SUPPORTED | Patch caching not supported |  |  |

| IOCTL_MIDI_CACHE_DRUM_PATCHES |  | Request an internal synthesizer preload a specified set of key-based percussion patches. Some synthesizers are not capable of keeping all percussion patches loaded simultaneously and must load data from disk when they receive MIDI program change messages. Caching drum patches ensures specified percussion patches are immediately available. The patch structure element specifies which drum patch number should be used. Specify 0 for the default drum patch. Each element of the patch array represents one of the 128 key-based percussion patches and has bits set for each of the 16 MIDI channels that use that particular patch. The least significant bit represents physical channel 15. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Input buffer size |  | >=sizeof(MIDI_DD_CACHE_DRUM_PATCHES) |  |  |  |
|  | Input buffer |  | MIDI_DD_CACHE_DRUM_PATCHES structure |  |  |  |
|  | Output buffer size |  | 0 |  |  |  |
|  | Output buffer |  | None |  |  |  |
|  | IOCB.Information |  | 0 |  |  |  |
|  | IOCB.Status |  | STATUS_SUCCESS | Percussion Patches cached |  |  |
|  |  |  | STATUS_BUFFER_TOO_SMALL | Not enough input data |  |  |
|  |  |  | STATUS_NOT_SUPPORTED | Percussion Patch caching not supported |  |  |