RAS 2.0 Netbios Gateway

Functional Specification

| Stefan Solomon | 05/07/92 | Created |
| --- | --- | --- |

Overview

This document presents the observable behaviour of Netbios calls over the RAS 2.0 Netbios gateway.

Netbios Names

The NCB.ADD.NAME operations at the remote client are intercepted by the gateway and the respective name is added on each LAN stack at the gateway.

If the name can not be added on a LAN stack, for whatever reason,  the following take place:

·	the name is deleted from all the LAN stacks at the gateway.

·	the gateway adds the name on the NBF async stack. This operation will result in a name conflict at the remote client , which will then  fail the NCB.ADD.NAME with the name_conflict error code.

·	the gateway keeps the name for 4secs on the NBF async stack,  to provoke the name conflict, and then deletes it.

The NCB.ADD.GROUP.NAME operation is completely similar with NCB.ADD.NAME.

The NCB.DELETE.NAME operation deletes the name on the remote client stack. The gateway gets the status of the remote client stack every 30secs. Using the data returned by the NCB.STATUS, the gateway  deletes the name on the LAN stacks. So, the name deletion is propagated at the gateway with a 30 secs delay (max).

Session Establishment

The NCB.CALL issued by the remote client is intercepted by the gateway. The gateway issues NCB.CALLs on all the gateway LAN stacks. The first completed call cancels the others. If the call gets completed, the gateway adds the server name on the NBF async stack (using the NCB.QUICK.ADD.NAME hook) and then posts a listen on this name to satisfy the client's NCB.CALL.

If the gateway remotelisten parameters is set, the gateway will post listen on all the clients names added to the LAN (or on all the message names, depending on the setting).

When such a listen completes, the gateway will try to call the remote client. If the remote client had a listen posted, a session is established and bridged with the one already established with the LAN caller.

Else, the session established on the LAN is hanged.

This behaviour is asymetrical, and can cause problems for apps which are based on NCB.CALL completion as an end-to-end indicator. The end indicator of this type is valid only for a remote client to LAN server call.

Session Hang-Up

A session between a remote client and a LAN server is hanged by the gateway when:

Either of the two ends hangs the session.

A network problem is detected on either end (Async or LAN stack).

Unlike RAS 1.0, there will be no session disconnection when the gateway is congested. The gateway will solve congestion via flow control (see below).

Session Data Transfer

Session data transfer is transparent to the sender and the receiver.  End to end send semantics is not kept. NCB.SEND completion means data arrived at the gateway, not at the receiver. If the gateway becomes congested, it will not stop submitting receives, thus applying backpressure on the sender. This form of flow control works, however, only if the LAN sender timeout is 0 (no timeout). This should enable apps other than Lan Manager to transfer any size of data without app level acknowledgements.

Datagrams Transfer

·	Datagrams from Async to LAN.

Any datagram from Async to LAN is transferred immediately, without buffering.

·	Datagrams from LAN to Async.

These are grouped as follows:

·	Unicast datagrams:

These datagrams are buffered and the buffers submitted to transmission on the LAN. Up to 16 datagrams can be buffered.

·	Multicast datagrams:

These datagrams will also be divided as follows:

·	Datagrams on preffered names.

These datagrams are buffered and delivered with the same policy as for unicast datagrams.

·	Other multicast datagrams and broadcasts:

These are filtered using the gateway settings for the multicastforwardrate. Datagrams are forwarded every N secs. on this channel. Every datagram coming in betwwen is silently discarded.