Network Component Binding

David Hovel
Microsoft Corporation
October 21, 1992
Revision 1.2

µ1.	Introduction	3

1.1.	Purpose	3

1.2.	The Problem Statement	3

1.3.	Method	4

2.	Mechanics	4

2.1.	SProlog Interpreter	4

2.2.	Registry-based Information	5

2.2.1.	Types of Registry Information	5

2.2.2.	Product Registration Information	5

2.2.3.	Service Registration Information	6

2.3.	Component Types	6

2.4.	Binding Algorithm	7

3.	Component Abstraction	8

3.1.	Component Model	8

3.2.	Device Classes	9

3.3.	Implications of Device Classes	10

3.4.	Multiple Classes	10

3.5.	Built-in Rules	11

3.6.	Device Types	11

3.7.	Binding Types	12

3.7.1.	Basic Binding Behavior	12

3.7.2.	Binding Constraints	12

3.7.3.	Binding Requirements	13

3.7.4.	Formatting of Binding Information	13

4.	Fact Definitions	14

4.1.	Internal Form	14

4.1.1.	devClass	14

4.1.2.	devType	15

4.1.3.	devBind	15

4.1.4.	bindable	16

4.1.5.	block	16

4.1.6.	height	16

4.1.7.	present	17

4.1.8.	bindstring	17

4.2.	External Form	18

4.2.1.	type (REG_SZ)	19

4.2.2.	use (REG_SZ)	19

4.2.3.	class (REG_MULTI_SZ)	20

4.2.4.	bindable (REG_MULTI_SZ)	20

4.2.5.	bindform (REG_SZ)	21

4.2.6.	Review (REG_DWORD)	21

4.2.7.	Hidden (REG_DWORD)	22

4.2.8.	RawRules (REG_MULTI_SZ)	22

4.3.	Extending the Rule Set	22

4.4.	Using an Alternate Rule Set	23

5.	Results of the Binding Operation	24

5.1.	General Behavior	24

5.2.	Generated Data	24

5.3.	Dependency Handling	26

5.4.	Specific Dependencies	26

5.5.	Group Depencencies	26

5.6.	Static Dependencies	27

5.7.	Binding Review	27

6.	Overview of Small Prolog Syntax	27

6.1.	General Form	27

6.2.	Lists	28

6.3.	Comments	28

6.4.	Prolog Operations	28

7.	Debugging with Rules	29

7.1.	Using an Expert System	29

7.2.	The NCPA Rule Set: NCPARULE.SPR	30

7.3.	The NCPA Test Application	30

7.4.	The Command Line SProlog Interpreter	30

7.5.	Putting it All Together	30

8.	Registry Data Examples	31

8.1.	Software Registration Data	31

8.2.	Netcard Registration	33

8.3.	Service Registration	34

# 1.	Introduction

## 1.1.	Purpose

The purpose of this document is to detail the method by which intercomponent dependencies are established and maintained within the Windows NT network protocol tower.

While this may appear to be a closely scoped problem, there are other product ensembles to which this methodology could apply.  Wherever possible, the discussion is kept general and avoids network-specific buzzwords.

The "configuration tool" mentioned in this document is synonymous with the NT Network Control Panel Applet (NCPA).  There may be other "configuration tools" in the future, but this is the only component in NT Product One which exhibits the behavior discussed herein.

## 1.2.	The Problem Statement

For the networking software in a single machine to operate properly, several different pieces of software must be loaded, and they must be told the relationships they are to establish with other components.   These relationships are often called "bindings" or "plumbing".

The final result is a lattice of nodes and arcs.  The nodes are software components residing in memory; the arcs represent operating-system-dependent "open" operations performed from a "higher" node (further from the hardware) to a "lower" one (closer to the hardware).

The problem, then is this:

Given an ensemble of configured products, determine the complete set of desirable bindings, and instruct the products accordingly.

Solving this puzzle requires several pieces of information:

the set of products to be configured;

the types of products in this set;

the constraints applicable to the products and their bindings;

the possible bindings which could occur;

the proper means of informing each product about its determined bindings.

In Windows/NT, the final result of solving this puzzle is a set of value data items to be stored into the Configuration Registry.

## 1.3.	Method

The method employed is a rule-based system, implemented in a subset of the Prolog language known as "Small Prolog", and referred to as SProlog in this document.  The SProlog interpreter is written in standard 'C' and is fully portable.

The rules divide into two categories.  The first category contains rules which which are "preset", and comprise the algorithms used to arrive at conclusions.  The second category contains "facts" (simple rules) which are elucidated from the Windows NT Configuration Registry, and describe the hardware and software available from the machine in question.

There are several advantages to such a system; some of the most important are:

the external format of facts is text, and is similar to existing INI files;

the internal (converted) format of the facts is never exposed;

then internal format of the rules is simple text;

the rules (algorithms) used are not embedded directly in the NCPA's C++ code; instead, they are read in from the NCPA's resource fork or the Registry during operations.

The exact nature of the implementation is not important, and will not be exposed directly either to users or ISV/IHVs.

# 2.	Mechanics

## 2.1.	SProlog Interpreter

A Prolog interpreter is basically a database and a set of database search algorithms.   The facts (declarative information) and rules (algorithm descriptions) are added to the database, algorithms are run, and the results are extracted from the database.

The SProlog interpreter has two basic functions:

Consult: adds facts and rules to the internal database.  SProlog can consult a file or a text string.

Query: attempts to answer a question; there may be more than one answer (non-deterministic query).  Queries can come from files or text strings.

## 2.2.	Registry-based Information

### 2.2.1.	Types of Registry Information

When a product is installed, various information is added to the Registry.  Each networking product is represented by two distinct areas:

a software registration key under "HKEY_LOCAL_MACHINE\Software\<manufacturer>\<version>", or,  for network adapters, a netcard registration key under "HKEY_LOCAL_MACHINE\Software\Windows NT\CurrentVersion\NetworkCards"; and

a service registration key under "HKEY_LOCAL_MACHINE\System\CurrentControlSet\Services".

(A complete example of the Registry data for a configured system is presented at the end of this document.)

### 2.2.2.	Product Registration Information

The software registration or netcard registration area contains several values which describe the product and allow it to be configured by the Network Control Panel Applet.  For networking products, a special key is created in the software or netcard registration area called NetRules.

The NetRules key serves two purposes.  First, it identifies the product as part of the network ensemble (in other words, this key is the "trigger" that causes the Network Control Panel Applet to include it).   Second, the values under it are converted to SProlog facts by the NCPA.  Using an 3Com Etherlink II network card as an example, the value item:

type=elnkii elnkiiAdapter

is converted to the declarative fact:

(devType elnkii adapter elnkiiAdapter elnkiiAdapter)

The installation program would only be responsible for creating the NetRules key and adding the proper values.  The conversion and use of the data is entirely the responsibility of the Network Control Panel Applet (NCPA).

A later section of this document defines the external and internal formats of the information.

It should be noted that the information obtained from software and hardware components can also be used to define entirely new types of components, since the binding rules used by the NCPA have no hard-coded component knowledge.

### 2.2.3.	Service Registration Information

The service registration area is used by the Service Controller to load the product into memory.   Thus, the Service Controller requires certain information, such as the location of the executable file and the service type and start criterion.

Each software product's registration information contains a value called ServiceName, whose data is the name of the service corresponding to the product.  It acts as a symbolic link to the service parameters used by the Service Controller.

Some products are actually sets of services, each of which has its own key in the services area; these keys are created (directly or indirectly) though the use of the Win32 Service Controller API.   In these cases, there is usually a "main" service, which is actively manipulated by the NCPA during bindings generation; the other services are listed as dependencies on the main service.  They are thus loaded by the Service Controller when the main service is started.

Each netcard is also represented in the services area.   The Service Controller ignores such entries, however, since they represent hardware objects and contain physical parameters of the network card, such as its I/O address and IRQ number.

Netcard service entries are created by direct use of the Win32 Registry APIs.

## 2.3.	Component Types

The only predefined terms known to the interpreter are:

adapter: a piece of hardware;

driver: a software component which is associated directly with a piece of hardware;

transport: a software component which is used by services;

service: a software component providing capability directly to user applications;

basic: a token used to represent a fundamental class name (that is, a class with no parent).

While these names are familiar to most network developers, no particular significance is given to them by the NCPA.  For example, a transport can be bound (connected) directly to an adapter.  This is not how NT LANMan components behave, but there is no implicit restriction which prohibits it.

Components found in the \Windows NT\CurrentVersion\Netcards section of the Registry are, by definition, adapters.  Components found in the the Software section are either drivers, transports or services, and must indicate their type.

Note that this implies that the installation of a single netcard will normally result in the creation of four distinct areas in the Registry:

the netcard registration key for the network card;

the software registration key for the driver;

the service registration key for the network card itself;

the service registration key for the driver.

The installation program would usually detect the fact that the driver or another instance of the same type of netcard is already present; in this case, only the Registry keys associated with the network card would be added.  See the chapter "Registry Data Examples" for detailed information.

The distinction between a transport and a service is somewhat subtle.  The important point is that a service is self-justifying; its mere presence indicates that every attempt should be made to connect it properly into the protocol tower.  A transport, on the other hand, is an intermdiary between services and drivers.  A transport or driver will not be loaded (and no error will be reported) if nothing requires (connects to) it.

## 2.4.	Binding Algorithm

As stated earlier, the SProlog engine is given one or more text files or strings to "consult".  Consultation adds the algorithms as "rules" into its database.

The file containing the algorithms are provided by Microsoft as part of the configuration tool (NCPA.CPL).  These rules can be changed by re-releasing the NCPA.  (Provision have been made for overriding these rules with a disk file, but this function is currently deactivated.)

After the SProlog interpreter has been "queried" to generate the binding data, the desired information is stored into its database.  In Windows/NT, this information consists of text strings describing objects in the NT name space.    For example:

\Device\Nbf_Elnkii01

might be the name of a 3Com Etherlink II card as "viewed" through (serviced by) the Netbeui transport.

As the Services area of the Registry is enumerated, the SProlog interpreter is queried as to the binding information applicable to the current service.  If any is found, a Linkage subkey is created, and the strings are stored.  For example:

<..>\Services\LanmanWorkstation\Linkage
	Bind = REG_MULTI_SZ  \Device\Nbf_Elnkii01   \
                           \Device\Nbf_Elnkii02

describes the binding information necessary for the NT LAN Manager Redirector when two separate netcards are present.  Each netcard's symbolic name is suffixed with a network card index number.  This name is joined to the name of the transport through which the network card is accessed.

The name strings are generated by the NCPA's binding algorithm according to the constraints defined by the product's rules.

Full information about the results of the binding algorithm can be found in a later chapter.

# 3.	Component Abstraction

Necessarily, the rules and facts governing component binding are based upon a model describing the characteristics of a generic component.  The model is abstract enough to encompass almost any type of device or software component.

This section describes the model used in the SProlog rules.  For convenience, this model is referred to as the binding model.

It is not strictly necessary that this chapter be fully understood; it is presented primarily for developers of complex or unusual software configuration which cannot be configured in ways similar to standard NT network models.

It is important to remember that installation software will only be responsible for writing a small number of values into the Registry under the new product's NetRules key. The conversion to SProlog format and the application of the binding model is entirely handled by the NCPA.

## 3.1.	Component Model

A software or hardware component can have either a top layer, a bottom layer or both.  An adapter card is usually viewed as having only a top layer, since it connects directly to the hardware.   Conversely, a redirector can be considered as having only a bottom layer, since it connects to network services, but no network services connect to it.

The binding model views every component as having both a top and bottom layer.  The layers may be of the same type, which is the default or normal case, or of different types.  For example, a typical network transport component binds to a driver and is bound by network applications layer providers.  However, some protocol stacks are referred to as "monolithic" because they span the functions of both a transport and a driver.

The binding model allows the component vendor to describe the behavior of the component's upper and lower layers separately.  This is unnecessary for typical components since these will be the same.

In the following example diagram, a Redirector is connected to two different protocol stacks.  The two stacks span different levels within the protocol tower.  In a case such as this, the Redirector, Netbeui and NDIS Driver would all be defined as being of the same class on both the top and bottom layers.  In other words, they span only one level of the protocol stack.   The Monolithic stack, on the other hand, behaves as a transport at its top and as a driver at its bottom.  This would usually be resolved by defining its top and bottom behavior as separate classes.

## 3.2.	Device Classes

Every component must be declared to be a member of at least one known class.  Bindings are generated when a rule allows a connection between one class and another, and an instance of each of the two classes is found.

Classes are simply convenient textual names; they must be unique.  Each class must either be a subclass (descendent) of a previously defined class, or must be declared as basic.   For example:

(devClass elnkiiAdapter basic no)
(devClass ndisDriver basic no)
(bindable ndisDriver elnkiiAdapter no)

These SProlog statements define two classes and a mutual binding rule.  Both classes are declared as basic, which means that they are not derived from any other classes.

As a further example, consider:

(devClass ieeeNetDriver ndisDriver no)

If a device is defined to be an ieeeNetDriver (that is, one which utilizes standard IEEE addressing) it can be bound to an elnkiiAdapter because ieeeNetDriver is a subclass (descendent) of ndisDriver, and there is a binding rule which allows an ndisDriver to connect to an elnkiiAdapter.

Why is the class approach used?  In the example above, if a network transport component could use any NDIS-compliant network driver, then a bind rule would state this fact, and any such transport would be bound to components of class ndisDriver or ieeeNetDriver.   On the other hand, if the transport required features present only in an ieeeNetDriver (hence, absent from an ndisDriver), its binding rule would restrict it from using any other type of driver.

Device classes can "multiply inherit";  that is, it is permissible to declare a class which is a subclass of more than one distinct class.

## 3.3.	Implications of Device Classes

Declaring a device as a member of a class is equivalent to stating that it conforms to and supports every aspect of the behavior of that class.  Similarly, declaring a new class which is derived from an existing class is equivalent to stating that members of the new class obey all of the base class's behaviors, and add some new characteristics not found in the original class.

## 3.4.	Multiple Classes

A  new class can be declared be a subclass of more than one parent class.  This flexibility allows generalized software more latitude in configuration options.

Consider a driver which can be connected to transports conforming to two distinct behaviors (it is assumed that the driver can distinguish the two behavioral types correctly at run time).   The following rules define a driver which can be connected to NDIS 3.0- or NDIS 2.0-compliant transports:

(devClass ndis30Driver basic no)
(devClass ndis20Driver basic no)
(devClass ndisBothDriver ndis30Driver no)
(devClass ndisBothDriver ndis20Driver no)
(devClass ndis30Transport basic no)
(devClass ndis20Transport basic no)
(bindable ndis30Transport ndis3 0Driver ...)
(bindable ndis20Transport ndis20Driver ...)
(devType easyGoingDriver driver ndisBothDriver ...)
(devType netbeui service ndis30Transport ...)
(devType tcpip service ndis20Transport ...)

Here, the class ndisBothDriver is defined as a subclass of both ndis30Driver and ndis20Driver.  Thus, bindings are permitted to easyGoingDriver from both netbeui and tcpip.

The declarative rule format will be explained in more detail later.

## 3.5.	Built-in Rules

The basic rule set defines certain fundamental component classes and their bindability rules.  These are the default rules:

/*	Generic device class declarations  */

(devClass ndisDriver       basic no)
(devClass ndisTransport    basic no)
(devClass netBiosTransport ndisTransport no)
(devClass lmNetService	   basic no)

/*	Binding declarations  */

(bindable ndisTransport    ndisDriver        non non 100)
(bindable lmNetService	   netBiosTransport  non non 100)

Taken in order, the class rules define class for an NDIS-compliant driver, an NDIS-aware transport, a NetBIOS transport as a subclass of NDIS-aware transport, and finally an LANMan/NT service.

The binding declarations indicate that NDIS transports can bind to NDIS drivers in a reflexively non-exclusive manner, and that LANMan/NT services can bind to NDIS NetBIOS transports in the same way.

These concepts will be explained in more detail later in this document.

## 3.6.	Device Types

The general SProlog form for declaring the type of a device is:

(devType name type upperLayerClass lowerLayerClass)

The components of this declaration are as follows:

name:  this is a text string used to identify this particular component.  It must be in lower case and unique across all other components.

type:  this string must be either adapter, driver, transport or service.

upperLayerClass: is the class name best describing the behavior of the upper layer (top boundary) of the component.

lowerLayerClass: is the class name best describing the behavior of the lower layer (bottom boundary) of the component.

## 3.7.	Binding Types

### 3.7.1.	Basic Binding Behavior

Bindings are created when the following conditions are met:

a bind rule is located;

a component is found of the class specified as the "bind from" class, or of some class derived from it;

a component is found of the class specified as the "bind to" class, or of some class derived from it.;

the other constraints of the binding are satisfied.

Only one binding will occur between any pair of components, regardless of the number of applicable rules.

### 3.7.2.	Binding Constraints

The basic binding operation is merely class name matching.  Additional constraints can be placed upon the binding to handle more complex issues.

The general form of the rule which defines possible bindings is:

(bindable fromClassName toClassName 
		exclusive1 exclusive2 value)

The fromClassName and toClassName are the name of the classes participating in the binding, and their order indicates its direction.

The exclusive parameters indicate whether or not more than one binding can occur.  If exclusive1 is the token exclusive (as opposed to non), then the fromClassName entity cannot be bound to any other devices if this binding is enabled.  Likewise, if exclusive2 is exclusive, then the toClassName entity cannot receive any other bind requests if this binding is enabled.

The value parameter is an integer which indicates the relative worth or importance of this binding.   Since several different exclusive or non-exclusive bindings could be possible between two given products, this parameter allows the vendor to clearly state "preferences".  The larger number value of this parameter, the greater its importance.  In other words, all possible bindings between two products are generated and examined.  If any are in conflict, the single binding with the highest "value" is chosen over all the others.

### 3.7.3.	Binding Requirements

There is a general requirement that a binding be "useful".  This means that when followed to its end, the binding terminates at either an adapter (i.e., a physical device) or at a "logical end-point".  A "logical end-point" is simply a software component which manages all further interconnection infomation internally.

Any bindings which fail to meet this criterion are automatically discarded by the bindings generation algorithm.

The reason for this requirement is that it avoids the creation of bindings (and the resultant loading of software components) which can never be of actual use because they are inherently incomplete.

For example, a user might have a running network and then choose to remove the adapter card.  Without the "usability" restriction, the generated bindings rwould still connect the upper level components and prepare them for loading even though the network was entirely unusable.

### 3.7.4.	Formatting of Binding Information

Once it has been determined that a binding is possible, the necessary data must be constructed in a fashion interpretable by the product in question.  The devBind rule handles this; the format is:

(devBind name "objectName" receivesBindings 			  	      		appearsInBindings simpleOrContainer)

where:

name is the component name in question;

"objectName" is the name to be used as the object name for NT name space objects;

receivesBindings will either be yes or no; if yes, the NCPA will add the generated binding strings to the component's service Linkage key;

appearsInBindings will either be yes or no; if no, the name of this component will not be inserted into the binding string;  netcard drivers, for example, do not appear in bindings due to the operation of the NDIS layer;

simpleOrContainer will either be simple or container; if simple, underscores are used to delimit component names; if container, backslashes are use, and the component must be capable of creating container objects.

The basic model here is that a binding string starts with the characters "\Device\"; the remainder of the name is effectively a path through the protocol tower.  That is, each component in the connection path is represented by its object name.  Separation between the component names is controlled by the whether it is "simple" or "container".

# 4.	Fact Definitions

This section details the internal and external forms of the rules and facts of the binding system.   As previously stated, it is the task of the NCPA to convert from the external form to the internal from when reading from the Registry, and to convert from the internal form to the external form when modifying the Registry.

Developers only have to understand the external form and may safely ignore the internal form of the rules.

## 4.1.	Internal Form

The internal form of a rule or fact is the form in which the SProlog system expects to encounter it.  The syntax of this form is explained later.

### 4.1.1.	devClass

(devClass className parentClassName no)

The devClass rule defines a component class name and the parent class.  If there is no parent class, (i.e., the class is primitive to all others) the token 'basic' is used.  Each component  must be declared as a member of a known class.

A class name can be derived from more than one parent class; in this case, bindings can occur based upon either parent class.  If the class is a fundamental one, the token basic is used as the parent class name.

The final variable in the devClass rule is either no or yes.  Normally, a component will specify no (this is the default value in the external, or Registry form of this rule).

The use of yes defines the component as a "logical end-point". In this rule set, a "logical end-point" means that lower-level multiplexing is not visible to higher level components.  For example, if a transport is connected to several netcards and netcard drivers, it would normally export an object name for each separate lower-level device.  However, if the transport is doing internal routing or mapping, it may wish to be marked as a logical end-point.  This will cause it to export only a single name representing the transport as an atomic entity.   This feature is used by the TCP/IP and NBT services.

### 4.1.2.	devType

(devType productName productGenericType upperClass lowerClass)

A devType rule defines the classes to which a particular product belongs.  productName must be a unique atom.  productGenericType must be one of adapter, driver, transport or service.  upperClass and lowerClass must be names defined in devClass rules.

### 4.1.3.	devBind

(devBind productName objectName getsBindings appearsInBindings 				namingMethod)

A devBind rule defines how the actual binding strings to be stored into the Registry are constructed for a particular product.  The product name is the same unique token used in devType.  The objectName is a string which provides the name used in the NT namespace.  Since more than one adapter card can be present, the rule set will automatically add a numeric identifying suffix to this string to guarantee uniqueness.

If getsBindings is yes, bindings strings are generated and stored into the Services area of the Registry for this item.  If appearsInBindings is yes, the object name of this component is concatenated onto the NT device path name.

The token namingMethod must be simple, container, or streams.  "Simple" names contain no imbedded backslashes, and therefore do not require that the NT Object Manager be used to create "container" objects.  "Container" names use imbedded backslashes between components, which requires that the associated drivers utilize container objects.

The naming method called "streams" causes the word Streams to appear in front of the name immediately superior to it.  For example, if the actual path is NBT connecting to the NT Streams environment, the exported object name for this device would normally be:

\Device\Nbt\Streams

Since this does not conform to the normal Streams approach, the "streams" naming method is used, which results in:

\Device\Streams\Nbt

which is the actual device name.

### 4.1.4.	bindable

(bindable fromClassName toClassName exclusive1 
	exclusive2 value)

The elements and usage of this rule have been explained elsewhere.

### 4.1.5.	block

(block higherClassName lowerClassName)

This fact disallows connections between any two components which are instances of the the two named classes or , regardless of the separation between them in the protocol tower.

It is useful to contrast this with the bindable fact.  "bindable" allows two components which are immediately adjacent to each other to be connected.  As an example, consider:

service >> transport >> driver >> netcard

At each boundary (indicated by >>), a bindable rule must exist which allows the connection to be created.  Additional bindable rules describing other possible configurations, such as a direct connection between the service and the driver are not relevant to this binding.

The block fact, however, can prevent a binding even if the two components are not adjacent.  For example, block could be used to prevent a service from using a netcard of a particular class.

The block fact is directional; that is, a fact such as:

(block oneThing anotherThing)

prevents instances of class oneThing from utilizing anotherThing, but not the reverse.  Two block facts would be necessary to accomplish this.

### 4.1.6.	height

(height productName stackHeight bindHeight)

The height fact is currently unimplemented.  In the future, it will be used to resolve complex linkage situations where several different vertical combinations of components could be created.   For example, if component A and B could bind to C or to each other, there are three possible results:  (A->B, B->C), (B->A, A->C) or (A->C, B->C).

The height fact contains two values. The first, stackHeight, is treated as a relative number indicating the products preferred stance in the protocol tower.  The second, bindHeight, indicates the product level to which the component prefers to bind.

Thus, to force particular configuration, different weights would be used:

(A->B, B->C)
	(height A 50 0) /* bindHeight is not relevant */
	(height B 40 0)
	(height C 30 0)

(B->A, A->C)
	(height A 40 0) /* bindHeight is not relevant */
	(height B 50 0)
	(height C 30 0)

(A->C, B->C)
	(height A 50 30) 
	(height B 40 30)
	(height C 30 0)

In the last example, the possible binding (A->B) is rejected in favor of (A->C) because C's height is closer to A's desired bind height.  The same occurs with (B->A), rejected in favor of (B->C).

Again, this functionality is not currently implemented.

### 4.1.7.	present

(present productId productName objectName registryName)

The present rule is generated by the NCPA based upon information derived from the Registry.

productId is the unique token used to describe the product in the rule sets.  It is the same as productName, except where multiple instances of the same product occur; the only case of this is multiple hardware adapters of the same type.

productName is the unique token used to describe the product itself.

objectName is a string which contains the name the product will create (or which will be created on its behalf) in the NT object namespace.

registryName is the name of the key which represents the product.

### 4.1.8.	bindstring

(bindstring productName RouteList BindString ExportString)

The bindstring rule is generated internally by a later stage of the binding algorithm, and contains  the unique component name, a list of name tokens indicating the full route through the protocol tower (RouteList), and the generated Windows NT object name string (BindString). and the name which is to be created to allow connection to this object as viewed by components which wish to connect to productName (ExportString).

This information will be explained in more detail later in this document.

## 4.2.	External Form

This section details the form in which facts are presented to the NCPA.   All of the information in this section will be found attached as value items to the NetRules key in registration areas of the Configuration Registry.  (A later chapter of this document has a complete Registry example.)

The NCPA reads these items, converts them from the external form to the internal form, and peforms the binding algorithm.  All network component information is gathered into a single aggregation of data; therefore, facts declared anywhere in the Registry can affect any other network component.

The items are presented separately from their internal forms because there is not always a one-to-one correspondance between them.

The descriptions in this section are presented as:

<value item name> = <value item data>

This is for simplicity, and for easy comparison to a Windows-style INI file.  In reality, there is no "equal" sign.

All these data items are of Registry type REG_SZ or REG_MULTI_SZ.  In other words, those marked as REG_SZ can have only a single value in a given component; those marked as REG_MULTI_SZ can have as many values as necessary.

Because the Registry information in being converted into Prolog  rules, the following restrictions apply to all user-definable names:

they may not begin with an upper-case letter, number or underscore;

remaining characters follow standard 'C' variable name limitations (e.g., no '$', '%', no characters greater than 0x7F or less than 'A');

quoted strings are not allowed;

names may not be longer than 30 characters.

These restrictions do not apply to object names, which should be quoted strings.

As should be apparent, no attempt is made to allow internationalization of this information, since none of it is ever exposed to users.  The SProlog interpreter uses 8-bit characters exclusively.

It is critically important that an effort be made to make user-definable names unique.  If name collisions occur, the NCPA's binding algorithm will generate unpredictable results.

### 4.2.1.	type (REG_SZ)

type = <product name> <class> [<lower class>]

The type value defines the type of the component in terms of abstract product classes.  If the optional lower class name is absent, the first (or upper level) class type name will be used for both its upper and lower classes.

This value is required for software products and netcards.

This value, along with the use value defined in the next section are used to create the devType fact.

### 4.2.2.	use (REG_SZ)

use = {service|driver|transport|adapter} {yes|no} {yes|no}

This data item only appears for software items.  It defines the role played by the component.  If absent, service is assumed.

A hardware device is automatically assumed to be an adapter.  Each software product may identify itself as a driver, transport or service to clarify its role in the protocol tower.  This distinction is used as follows:

A service provides end-user functionality, and every attempt is made to support its operation.   A EventLog entry is generated if a service is present in the system for which there is no available transport (number of generated possible bindings is zero).

A transport, on the other hand, exists only to support services.  If no bindings are generated (or permitted by the user) which include a particular transport, that transport is not loaded.  However, no error is generated, since no "denial of service" has occured.

A driver exists only to support one or more adapters.  It, like a transport, is not loaded unless necessary.

The final two items are optional; if present, each must be either yes or no.  The first item indicates whether driver group names are used in lieu of specific driver dependencies.  The second indicates whether transport group names are used in lieu of specific transport dependencies.

For example, the LanmanServer is marked as "yes yes"; this means that its dependencies upon transports and drivers are at the group level; thus, LanmanServer will be loaded if any one of its transport dependencies and any one of its driver dependencies successfully load.

These indicators cause the NCPA to generate references to dependencies based upon their group names, not by their specific service names.  For more information on dependency handling, refer to the chapter on result of the binding algorithm; for more information on groups and group handling, refer to the Win32 Service Controller documentation.

This value, along with the type value defined in the previous section are used to create the devType fact.

### 4.2.3.	class (REG_MULTI_SZ)

class = <new class name> {<existing class name>|basic} [{no|yes]]

The class item allows a component to define a new class.  Since it is a REG_MULTI_SZ, as many new classes as are necessary may be defined by any component.

Class names do not need to be defined within any particular component.  The NCPA adds the new definition to its database without regard to origin.  The order of class items is irrelevant.  However, result are indeterminate if classes are referred to which are not defined anywhere in the entire rule ensemble.

This item is optional, since there are a few predefined classes, and class definition made anywhere in the product ensemble apply to all products.  Since any product may define new classes, care should be taken that the names used are unique within all possible installable products.  In other words, the NCPA converts the information into rules and applies all rules at once, regardless of origin.

The optional token indicates whether this class is a "logical end-point"; the default value is no.

### 4.2.4.	bindable (REG_MULTI_SZ)

bindable = <fromClass> <toClass> {yes|no} {yes|no} 					<value>

The bindable item defines a possible binding and its constraints.  Its elements correspond on a one-to-one basis with those of the internal form of the bindable rule.  (See documentation for bindable rule for details.)   For example:

bindable = ndisDriver ndisAdapter non exclusive 100

This value item specifies that components of class ndisDriver can be bound to those of class ndisAdapter.  The token non indicates that the component of class ndisDriver can accept other bindings; the token exclusive indicates that the component of class ndisAdapter cannot accept other bindings.   The number 100 indicates the relative importance (weight) of this binding; that is, in cases of competition, it will be discarded in favor of other bindings whose weight is greater.

Because this value item is a REG_MULTI_SZ, as many bindability criteria as necessary may be declared by a single component.

This item is optional, since there are a few predefined bindable rules, and bindable rules defined anywhere in the product ensemble apply to all product classes.

### 4.2.5.	bindform (REG_SZ)

bindform = <objectName> {yes|no} {yes|no} 		     	   		    {container|simple|streams}

The bindform item corresponds to the devBind fact.

The objectName field contains the name (or name prefix) by which the component will be identified in the NT object name space.  NOTE: In NT version 1, this value must be the same as the service name.  Object names for adapters are generated by the NCPA and override the "bindform" setting.

The first yes/no pair indicate whether the component is to receive binding information directly in its Linkage subkey.  The second yes/no pair indicates whether the device name is supposed to appear in generated binding strings.  Refer to the documentation for the internal devBind rule for more information.

The final token in the value is optional, and indicates how binding string device names are constructed.   See the earlier description of bindform for more details.

This value is required for software components.

### 4.2.6.	Review (REG_DWORD)

review  = { 0 | 1 }

The review item does not correspond to a fact.  It is used to indicate whether or not a component requests bindings review.  If set to "1" (or merely non-zero), the NCPA will reinvoke this component's INF file after bindings have been changed.  This allows products to modify the binding information or request additional information from administrators about the new or altered connections.

This value is a REG_DWORD.

See the chapter on results of the binding operation for more details on this item.

### 4.2.7.	Hidden (REG_DWORD)

hidden  = { 0 | 1 }

The hidden item does not correspond to a fact.  It is used to suppress the display of the component (adapter or software product) by the NCPA.

Normally, all networking components discovered in the Registry are displayed in the two listboxes on the expanded form of the NCPA's dialog.

Setting a product to hidden=1 simply prevents the item from being displayed, and hence, configurable or removable.

### 4.2.8.	RawRules (REG_MULTI_SZ)

The RawRules value item is different from all the others just presented.  It allows direct interaction with the inference engine through unrestricted rule addition.

Its format is:

RawRules = (rule 1 data)
		(rule 2 data)
		...

The NCPA will simply combine all RawRules with the generated configuration rule set before running the inference engine.

This feature is entirely "caveat emptor": it will not be directly supported, and should be avoided unless absolutely necessary.

## 4.3.	Extending the Rule Set

NOTE: If you do not understand the concepts mentioned in this section, do not attempt to use this feature.

The RawRules value documented above can be used to modify the binding information generation algorithm.  This is possible because the basic predicate queried by the NCPA can be "hooked" by rules added through RawRules values.

The (makebindstrings) predicate is the outermost level predictate called by the NCPA.  It is similar to the following:

( (makebindstrings)
    (not (userExtensions createbindings))
    (createbindings)    
    (not (userExtensions pruneexclusive))
    (prunexclusive)
    (not (userExtensions allbindstrings))
    (allbindstrings)
    (not (userExtensions pruneblocked))
    (pruneblocked)
    (not (userExtensions endofquery))
    (tracestat)
)

Before each of its steps, it invokes a predicate called userExtensions of arity 1 which is expected to fail, and, hence, is enclosed in the (not (X)) predicate.  The userExtensions predicate is expected to fail because it does not exist in the standard rule set.

To add a "user extension", you must define a userExtensions predicate of arity 1 and include it in a RawRules value.  As the example above shows, the makebindstrings predicate will pass the name of the upcoming phase to the userExtensions predicate.  This predicate may do anything necessary to modify the information at this phase, but two things are critically important:

·	it must fail; and

·	it should not generate backtrack points.

Careful analysis of the NCPA's rule set is necessary to meet the conditions and not destabilize the SProlog query.   The commented form of the NCPA's rule set explains the algorithm in detail.

Other qiueries made by the NCPA are solely for the purpose of extracting information from the SProlog database by backtracking; they invoke no rules.

## 4.4.	Using an Alternate Rule Set

There are possible situations when the internal, built-in rule set used by the NCPA is inadequate or incorrect.  There is a method of substituting a new rule set without recompilation of the NCPA.

Normally, the internal rule set is stored in the executable file NCPA.CP:L as a Windows "resource" named NCPARULE.

However, a value named Rulefile can be added to the NCPA's software registration area. It has the following form:

Rulefile (REG_SZ) = <name of ASCII text file>

If present, the file named by this value is opened, and its contents are used in lieu of the default values.

This feature is entirely "caveat emptor": it will not be directly supported, and should be avoided unless absolutely necessary.

# 5.	Results of the Binding Operation

## 5.1.	General Behavior

Each component whose presence is discovered to be necessary by the binding algorithm can be given bindings.  Whether the bindings actually appear in the Registry depends upon the bindform rule for the component.

The binding information is written to the Linkage key in three different forms, each of which is a REG_MULTI_SZ:

Bind:  each string in this value is the name of an NT object created by the underlying software.  (This is not strictly true, but is the general scheme.)

Export:  each string in this value indicates the name which should be added to the NT name space to allow access to the corresponding bound object;

Route: each string in this value indicates the exact path through the protocol tower represented by the binding; the names of services are used, surrounded by quotation marks.

Each element of these REG_MULTI_SZ values has a one-to-one correspondance with the others; that is, bind[1] is to export[1] and route[1] as bind[2] is to export[2] and route[2].

In addition to generating binding information, the NCPA's binding algorithm determines each product's complete set of dependencies, and stores them into the Registry through the use of the Service Controller APIs.

## 5.2.	Generated Data

Here is an example which uses NBF.SYS and SRV.SYS (assuming they are marked as "container") in an ensemble with three network cards, two Etherlink IIs and an IBM Token Ring card:

\Services\Nbf\Linkage

Bind   = "\Device\ElnkII1"

"\Device\ElnkII2"

"\Device\IbmTok1"

Export = "\Device\Nbf\ElnkII1"

"\Device\Nbf\ElnkII2"

"\Device\Nbf\IbmTok1"

Route  = "ElnkIISys ElnkII1"

"ElnkIISys ElnkII2"

"IbmtokSys IbmTok1"

\Services\Srv\Linkage

Bind   = "\Device\Nbf\ElnkII1"

"\Device\Nbf\ElnkII2"

"\Device\Nbf\IbmTok1"

Export = "\Device\Srv\Nbf\ElnkII1"

"\Device\Srv\Nbf\ElnkII2"

"\Device\Srv\Nbf\IbmTok1"

Route  = "Nbf ElnkIISys ElnkII1"

"Nbf ElnkIISys ElnkII2"

"Nbf IbmtokSys IbmTok1"

The names used in the Bind and Export strings are based upon the object names declared by the component in its NetRules (as stated earlier); they can therefore be different from the actual names of the services, although in this example, for the sake of clarity, they are not.

The names used in the Route strings are the names of the Service keys comprising the full downward route through the protocol tower.

Alternatively, if NBF.SYS were marked as requiring "simple" names, the data would appear as:

\Services\Nbf\Linkage

Bind   = "\Device\ElnkII1"

"\Device\ElnkII2"

"\Device\IbmTok1"

Export = "\Device\Nbf_ElnkII1"

"\Device\Nbf_ElnkII2"

"\Device\Nbf_IbmTok1"

Route  = "ElnkIISys ElnkII1"

"ElnkIISys ElnkII2"

"IbmtokSys IbmTok1"

\Services\Srv\Linkage

Bind   = "\Device\Nbf_ElnkII1"

"\Device\Nbf_ElnkII2"

"\Device\Nbf_IbmTok1"

Export = "\Device\Srv\Nbf_ElnkII1"

"\Device\Srv\Nbf_ElnkII2"

"\Device\Srv\Nbf_IbmTok1"

Route  = "Nbf ElnkIISys ElnkII1"

"Nbf ElnkIISys ElnkII2"

"Nbf IbmtokSys IbmTok1"

In this case, NBF's exports do not require container objects.

Note that the Route information remains the same, since it is comprised only of service names.  This information will be used by the RPC mapping layer for protocol string handling and by the NCPA for maintaining persistent binding information.

If a component does not handle exports, it can merely ignore the information.

## 5.3.	Dependency Handling

The Win32 Service Controller supports the concept of dependent services;  in other words, services can be marked as being dependent upon other services or drivers, which, in turn, can be dependent upon others, and so on.

Two types of dependencies are allowed:

specific dependencies, which are represented by the names of the services upon which the service in question is dependent; and

group dependencies, which are symbolic names listed in the ServiceGroupOrderList key.

The NCPA handles computation and establishment of dependency information.  There are three types of dependency information considered by the NCPA.

Dependency names are listed in a REG_MULTI_SZ value item attached to the main service key of a service.

## 5.4.	Specific Dependencies

A specific dependency is simply the name of a necessary service.  By default, the NCPA will generates explicit names for all dependent services discovered during bindings generation.

As an example, assume LanmanWorkstation is dependent upon Nbf.  Nbf is connected to two adapter cards, and so is dependent upon their drivers.  The NCPA will mark Nbf as dependent upon the two netcard drivers, and will mark LanmanWorkstation as dependent upon the netcard drivers and Nbf.

Typically, however, group dependencies are used for LANMan components.

## 5.5.	Group Depencencies

It often happens that a service should be loaded if any member of a set of dependencies successfully loads.   In the previous example, the LanmanWorkstation would fail to load if either of the netcard drivers failed to initialize.

Groups are used to support this approach.  Any service (driver, transport or whatever) can identify itself as being a member of a service group.  All NT network card drivers, for example, are treated as members of group NDIS.

Products can state whether their dependencies should be treated as specific or group-based by using the final two parameters of the "use" value.

## 5.6.	Static Dependencies

A "static dependency" is a service which is required in all circumstances, and which is unrelated to the results of the binding algorithm.

Since the NCPA will recompute dependencies, it also discards any previously listed dependencies.   In order to guarantee that a service is always configured to be dependent upon another service, the value item OtherDependencies can be created under the Linkage key.

OtherDependencies is a REG_MULTI_SZ, and can contain the names of as many services as needed.  See the listings under LanmanWorkstation in the chapter entitled "Registry Data Examples".

## 5.7.	Binding Review

When the bindings algorithm is completed and all of its results (including dependencies) are stored into the Registry, some software products may need to be informed of the changes which have occurred.

For example, TCP/IP needs to be able to ask the user for an IP address for any network adapter which it has been newly configured to use.

If the software registration area of a product has a REG_DWORD value item called Review set to a non-zero value, the INF file for the product will be invoked every time the bindings have been changed.

See the document NCPA.DOC for more details.

# 6.	Overview of Small Prolog Syntax

## 6.1.	General Form

To facilitate efficient parsing, SProlog uses a LISP-like syntax.  All statements are bounded by a balanced number of parentheses.

SProlog lexical tokens are:

atoms: words without intervening blanks which start with a lower-case letter;

variables:  words without intervening blanks that start with an upper case letter, or the underscore character standing alone;

strings: characters enclosed in double quotes;

integers: contiguous decimal digits, possibly preceded by a minus sign;

real numbers:  strings of characters in Fortran-G scientific notation, such as -1.45E-10.

An example SProlog statement might be:

(weather seattle rainy 310)

Note that seattle is not capitalized, because doing so would cause it to be interpreted as a free (unbound) variable.

## 6.2.	Lists

Every SProlog statement is really a list.  Lists are defined by enclosing a series of zero or more lexical tokens in parentheses.  Thus, the empty list is signified by ().  The vertical bar is used to indicate the tail of a list.  For example:

(HeadOfList | TailOfList)

This is a common set of tokens in SProlog, since list manipulation is one of Prolog's most powerful features.

## 6.3.	Comments

SProlog allows standard 'C'-style comments:

(this is not a comment)
/*  This is a comment  */
(this is also not a comment)

## 6.4.	Prolog Operations

Prolog works by pattern matching.  When the question mark prompt appears in the command line version of SProlog, the user may type a Prolog statement.  Prolog then attempts to "resolve" the statement by finding a matching statement in its database.  One successful result is returned for each match found (non-determinism).

For example, assume the data base contained:

(weather seattle rainy 310)
(weather sandiego sunny 200)
(weather nome snowy 250)

Then, if the user typed:

(weather City Mostly _)

SProlog would respond:

City = seattle, Mostly = rainy.
City = sandiego, Mostly = sunny.
City = nome, Mostly = snowy.

Since City and Mostly are capitalized, they are "free variables", and can take on any value found.  The underscore character indicates a "unnamed variable" and is ignored.

If the user typed:

(weather seattle sunny _)

Sprolog would respond:

No.

If the user typed:

(weather seattle rainy _)

Sprolog would respond:

Yes.

Those wishing more information about SProlog can obtain the command line version of the interpreter from the author of this document.  It runs on OS/2, NT and DOS.

Those wishing more information about Prolog are encouraged to refer to Programming in Prolog, by Clocksin and Mellish.

# 7.	Debugging with Rules

## 7.1.	Using an Expert System

The expert system contained in the Network Control Panel Applet will only function successfully if the information it is given is correct and complete.  If errors occur, the most common visible behavior is a complete failure of a component to bind to anything.

This can be frustrating, and the causes are not always readily apparent.  While the NCPA will (eventually) emit Win32 "event log" entries attempting to describe the problem, this information will probably suffice only to narrow the scope of the problem, not resolve it.

There are three key parts to the NCPA's expert system:

the SProlog interpreter;

the basic rule set, which is contained in NCPA.CPL as a user-defined resource;

the derived rule set, which NCPA.CPL generates by converting Registry-based information into SProlog rules.

Full-scale problem diagnosis occasionally requires access to all three of these elements.  This section describes how to obtain and use these elements.

This section and the tools it describes assume some familiarity with the basic concepts of Prolog and the basic syntax of Small Prolog.

## 7.2.	The NCPA Rule Set: NCPARULE.SPR

The basic rule set used by the NCPA is kept in a text file called NCPARULE.SPR, which is in the XLATE directory of the NCPA project.  This file is scanned by RCPP to remove comments, and then stored by the Win32 Resource Compiler into the .RES file, and bound into NCPA.CPL.

## 7.3.	The NCPA Test Application

The NCPA source tree has a directory called "TEST" which builds a Win32 application called NCPAPP.EXE.

NCPAPP has an empty main window with a standard menu bar.  Choosing "Run", for example, causes NCPAPP to execute the same code which is contained in NCPA.CPL.

Chosing "Test" and then "Facts" brings up a dialog containing an edit control filled with the SProlog facts generated by scanning the Registry.  These are the same facts which NCPA.CPL would add to the basic rules.  At the same time that NCPAPP products the "Facts" dialog, it writes the facts to a text file called REGFACTS.SPR.

## 7.4.	The Command Line SProlog Interpreter

Another directory in the NCPA project is "SPTEST"; building in this directory produces a Win32 command-window application called PRMAIN.EXE.   This program runs in a simple TTY mode, similar to many other LISP-ish interpreters.   It is very similar to the basic Small Prolog application as obtained from public domain sources.

## 7.5.	Putting it All Together

Combining these elements allows you perform the same rule analysis which the NCPA.CPL performs, but it allows you to monitor, alter and debug problems easily.

The steps are:

run NCPAPP, menu item Test/Facts, to produce REGFACTS.SPR;

run PRMAIN.EXE to reproduce the rule analysis.

Here is a sample session:

prmain /n ncparule.spr         {run PRMAIN.EXE}
?- (consult "regfacts.spr")    {facts from NCPAPP}
?- (makebindstrings)           {run bindings algorithm}
?- (bindstring Product List Str Expstr)
?- (quit)                      {leave the interpreter}

The "?-" characters are prompts emitted by the PRMAIN application.  The "/n" option tells PRMAIN not to consult the default rule set; this is important because the default rules are already contained in NCPARULE.SPR.  Consulting them twice would cause failure of the interpreter.

The line (consult regfacts.spr) causes PRMAIN to read in the facts generated by NCPAPP.

The line (makebindstrings) runs the binding algortihm.  Its results are "asserted" into the internal Prolog database.

The final line (bindstring Product List Str Expstr) is a query which causes SProlog to enumerate each bind string triplet (bind string, export string and route string).   Replacing the variable atom Product with the atomic non-variable name of the specific product of interest (such as "elnkii") will result in just the binding information for that product.

# 8.	Registry Data Examples

This section contains the Registry data from a sample installation of the Windows NT SDK, released on CD-ROM on July 6, 1992.

The configured system had a standard NT LANMan protocol tower:  LanmanWorkstation, LanmanServer, Nbf (NetBEUI), Elnkii (the driver for the 3Com Etherlink II netcard), and the NetBIOS service.

The data is presented in HIVEDMP format.

## 8.1.	Software Registration Data

The information under "HKEY_LOCAL_MACHINE\Software\Microsoft" describes installed software products which belong to Microsoft Corp.

The NetRules key is the signal to the NCPA that the component in question is part of the network ensemble.

Microsoft

Elnkii

3.1

InfName = E:\winnt\system\OEMNADE2.INF

ServiceName = Elnkii

SoftwareType = driver

Title = 3Com Etherlink II Adapter Driver

Description = Elink II System Driver

PathName = E:\winnt\system\

InstallDate = REG_DWORD 0x2a4e01c5

NetRules

type = elnkiiSys ndisDriver elnkiiDriver

use = driver

bindform = "ElnkIISys" yes no container

class = REG_MULTI_SZ "elnkiiDriver basic"

bindable = REG_MULTI_SZ "elnkiiDriver 						elnkiiAdapter non exclusive 100"

InfOption = ELNKII

Infname = E:\winnt\system\OEMNADE2.INF

LanmanServer

3.1

InfName = E:\winnt\system\OEMNSVSV.INF

ServiceName = LanmanServer

SoftwareType = service

Title = Windows Server

Description = Microsoft Windows Server Service

PathName = E:\winnt\system\

InstallDate = REG_DWORD 0x2a4e01eb

NetRules

type = srv lmNetService

use = service yes yes

bindform = "Srv" yes yes container

InfOption = SRV

Infname = E:\winnt\system\OEMNSVSV.INF

LanmanWorkstation

3.1

InfName = E:\winnt\system\OEMNSVWK.INF

ServiceName = LanmanWorkstation

SoftwareType = service

Title = Windows Workstation

Description = Microsoft Windows Workstation 					Service

PathName = E:\winnt\system\

InstallDate = REG_DWORD 0x2a4e01fd

NetRules

type = wskta lmNetService

use = service yes yes

bindform = "Wksta" yes yes container

InfOption = WKSTA

Infname = E:\winnt\system\OEMNSVWK.INF

Nbf

3.1

InfName = E:\winnt\system\OEMNXPNB.INF

ServiceName = Nbf

SoftwareType = transport

Title = Microsoft NetBEUI

Description = Microsoft NetBEUI 3.0 Transport

PathName = E:\winnt\system\Nbf

InstallDate = 709755362

NetRules

type = nbf netBiosTransport

use = transport yes yes

bindform = "Nbf" yes yes simple

InfOption = NBF

Infname = E:\winnt\system\OEMNXPNB.INF

Ncpa

3.1

HeapSize = REG_DWORD 0x00030d40

StringsSize = REG_DWORD 0x0000c350

DynSize = REG_DWORD 0x0000c350

TrailSize = REG_DWORD 0x0000c350

SubstSize = REG_DWORD 0x0000c350

TempSize = REG_DWORD 0x0000c350

ReadBuffSize = REG_DWORD 0x000007d0

PrintBuffSize = REG_DWORD 0x000007d0

NtlanmanInfName = REG_EXPAND_SZ 							%systemroot%\SYSTEM\NTLANMAN.INF

NcpaShelInfName = REG_EXPAND_SZ 							%systemroot%\SYSTEM\NCPASHEL.INF

InstallSourcePath = F:\i386\

DomainEnable = REG_DWORD 0x00000000

Netbios

3.1

InfName = E:\winnt\system\OEMNSVNB.INF

ServiceName = Netbios

SoftwareType = service

Title = NetBIOS Interface

Description = Microsoft NetBIOS Transport 						Interface

PathName = E:\winnt\system\

InstallDate = REG_DWORD 0x2a4e021c

NetRules

type = netbios lmNetService

use = service yes yes

bindform = "Netbios" yes yes container

InfOption = NETBIOS

Infname = E:\winnt\system\OEMNSVNB.INF

## 8.2.	Netcard Registration

This section is located under "HKEY_LOCAL_MACHINE\Software\Microsoft\Windows NT\CurrentVersion".

The NCPA automatically includes all non-disabled adapters into the network ensemble.

NetworkCards

01

InfName = E:\winnt\system\OEMNADE2.INF

ServiceName = Elnkii01

Manufacturer = Microsoft

Title = 3Com Etherlink II Adapter

Description = Elink II Network Card

ProductName = Elnkii

InstallDate = REG_DWORD 0x2a4e01c5

NetRules

type = elnkii elnkiiAdapter

bindform = "Elnkii01" yes yes container

class = REG_MULTI_SZ "elnkiiAdapter basic"

InfOption = ELNKII

Infname = E:\winnt\system\OEMNADE2.INF

## 8.3.	Service Registration

Data in this section appears under "HKEY_LOCAL_MACHINE\System\CurrentControlSet\Services".

This information is used by the Service Controller to determine the load order of optional system components.

Elnkii

Type = REG_DWORD 0x00000001

Start = REG_DWORD 0x00000002

ErrorControl = REG_DWORD 0x00000001

ImagePath = REG_EXPAND_SZ 					  	                      \SystemRoot\system\drivers\elnkii.sys

Group = NDIS

Dependencies = REG_MULTI_SZ

Linkage

Bind = REG_MULTI_SZ "\Device\Elnkii01"

Export = REG_MULTI_SZ "\Device\Elnkii01"

Route = REG_MULTI_SZ ""Elnkii01""

Parameters

Elnkii01

Type = REG_DWORD 0x00000004

Start = REG_DWORD 0x00000003

ErrorControl = REG_DWORD 0x00000001

Linkage

Parameters

Transceiver = REG_DWORD 0x00000001

MemoryMapped = REG_DWORD 0x00000000

InterruptNumber = REG_DWORD 0x00000005

IoBaseAddress = REG_DWORD 0x00000310

LanmanServer

Type = REG_DWORD 0x00000020

Start = REG_DWORD 0x00000002

ErrorControl = REG_DWORD 0x00000001

ImagePath = REG_EXPAND_SZ %SystemRoot%\system\lmsvcs.exe

ObjectName = LocalSystem

Dependencies = REG_MULTI_SZ "Srv"

AutotunedParameters

Linkage

OtherDependencies = REG_MULTI_SZ "Srv"

Bind = REG_MULTI_SZ "\Device\Nbf_Elnkii01"

Export = REG_MULTI_SZ "\Device\Srv\Nbf_Elnkii01"

Route = REG_MULTI_SZ ""Nbf" "Elnkii" "Elnkii01""

Parameters

Size = REG_DWORD 0x00000002

Shares

LanmanWorkstation

Type = REG_DWORD 0x00000020

Start = REG_DWORD 0x00000002

ErrorControl = REG_DWORD 0x00000001

ImagePath = REG_EXPAND_SZ %SystemRoot%\system\lmsvcs.exe

ObjectName = LocalSystem

Dependencies = REG_MULTI_SZ "Rdr"  \

"Bowser"  \

"Mup"

Linkage

OtherDependencies = REG_MULTI_SZ "Rdr"  \

"Bowser"  \

"Mup"

Bind = REG_MULTI_SZ "\Device\Nbf_Elnkii01"

Export = REG_MULTI_SZ "\Device\Wksta\Nbf_Elnkii01"

Route = REG_MULTI_SZ ""Nbf" "Elnkii" "Elnkii01""

Parameters

Nbf

Type = REG_DWORD 0x00000001

Start = REG_DWORD 0x00000002

ErrorControl = REG_DWORD 0x00000001

ImagePath = REG_EXPAND_SZ                                                                                   	                      \SystemRoot\system\drivers\nbf.sys

Group = TDI

Dependencies = REG_MULTI_SZ "NDIS"

Linkage

OtherDependencies = REG_MULTI_SZ "NDIS"

Bind = REG_MULTI_SZ "\Device\Elnkii01"

Export = REG_MULTI_SZ "\Device\Nbf_Elnkii01"

Route = REG_MULTI_SZ ""Elnkii" "Elnkii01""

Parameters

Netbios

Type = REG_DWORD 0x00000002

Start = REG_DWORD 0x00000002

ErrorControl = REG_DWORD 0x00000001

ImagePath = REG_EXPAND_SZ 	  	 				                \SystemRoot\system\drivers\netbios.sys

Group = Network

Dependencies = REG_MULTI_SZ "TDI"

Linkage

OtherDependencies = REG_MULTI_SZ "TDI"

Bind = REG_MULTI_SZ "\Device\Nbf_Elnkii01"

Export = REG_MULTI_SZ "\Device\Netbios\Nbf_Elnkii01"

Route = REG_MULTI_SZ ""Nbf" "Elnkii" "Elnkii01""

Parameters