Portable Systems Group

Microsoft Tape Driver Specification

Author:  Steve Devos & Norbert P. Kusters

Revision 1.0, March 3, 1995

Do not remove any of the division marks (:::) in this template.  They control the basic layout of the document, including the way page numbers are printed.

1. Overview

2. Tapeminiclass Driver Initialization

2.1 Tapeminiclass DriverEntry Function

2.2 Specifying TapeminiclassExtensionSize

2.3 Specifying DefaultTimeOutValue

2.4 Specifying CommandExtensionSize

2.5 VerifyInquiry routine

3. ProcessCommand routines

3.1 General aspects of all ProcessCommand routines

3.1.1 Example:  SCSI Mode Sense request

3.1.2 Example:  SCSI Mode Select request

3.1.3 Example:  SCSI Rewind request

3.2 Descriptions for specific ProcessCommand routines

4. TapeError routine

5. TAPE_STATUS codes

6. Tape/Minitape interface definitions

7. MINITAPE.H

.Begin Table C.

.End Table C.

# Overview

Microsoft’s 32bit operating systems support for tape devices requires two drivers: The Tapeclass driver and the Tapeminiclass driver.  The Tapeclass driver is OS specific but device independant.   The Tapeminiclass driver is device specific but OS independant.

The Tapeminiclass driver is responsible for building and intrepreting SRBs and CDBs.  It contains the main entry point for the driver as well as several ProcessCommand routines (ex. WriteMarks, Erase, Prepare, CreatePartition, and GetPosition). Each of these ProcessCommand routines defines a descreet function.   Many of these functions require several SRBs.  To provide for this, these routines are call over and over again until either an error occurs or the operation is complete.  With each call, a CallNumber input parameter is incremented.   The Tapeminiclass driver entypoins use this CallNumber to maintain its state.

Since the Tapeminiclass driver is OS independant, it can not call OS specific functions.  The Tapeclass driver provides a small number of helper routines.  These routines include functions for allocating memory, initializing memory, and debug printing.  Two common tape specific conversion routins are also exported.  These two routines convert between QIC logical positions and physical positions.

The Tapeclass driver interfaces with the other drivers in the system to get the CDBs to the tape device. The funnctional entrypoints in the tape driver are contained in the Tapeclass driver.   These entrypoints use the Tapeminiclass to build and intialize these CDBs.  The Tapeclass driver uses a function table that was provided at initialization time in to call the Tapeminiclass driver. as a state machine.  After each call, the state is incremented.   The Tapeminiclass routines specify via thier return value what the Tapeclass driver should do next.   In many casses, the Tapeminiclass routine will request that a SRB be sent and that the Tapeminiclass routine should be called back.  On subsuquent call, the Tapeclass routine will increment the CallNumber parameter to the Tapeminiclass.

A Tapeminiclass driver writer should be familiar with the SCSI-2 command specification and how it relates to tape drives.

# Tapeminiclass Driver Initialization

At driver intialization time, the Tapeminiclass driver is called by the system.   The Tapeminiclass drive initializes a TAPE_INIT_DATA strucnture and passes it to the Tapeclass driver who is responsible for creating the device object.

## Tapeminiclass DriverEntry Function

The DriverEntry routine is the initial entry point for every Tapeminiclass driver.  A Tapeminiclass’s DriverEntry function (see section 7 for prototype) is called by the system with two arguments of type PVOID and must do the following:

- Put a TAPE_INIT_DATA structure on the stack.

- Set driver-specific and tape driver specific value in the TAPE_INIT_DATA members, including the Tapeclass driver’s entry points.  The following entry points must be set:

- VerifyInquiry

- CreatePartition

- Erase

- GetDriveParameters

- GetMediaParameters

- GetPosition

- GetStatus

- Prepare

- SetDriveParameters

- SetMediaParameters

- SetPosition

- WriteMarks

- The following entry points can be set to a driver-supplied function or must be set to NULL:

- ExtensionInit

- TapeError

- The following values can be set to a driver-supplied value or must be set to 0:

- TapeminiclassExtensionSize

- DefaultTimeOutValue

- CommandExtensionSize

- Call TapeClassInitialize (see Section 7 for prototype) with the pointers that were input to the DriverEntry function and the address of the filled-in TAPE_INIT_DATA.

- Return with the value returned from TapeClassInitialize.

## Specifying TapeminiclassExtensionSize

Each Tapeminiclass driver can define it’s own static storage for the purposes of saving context information on a per device basis.  The Tapeminiclass driver specifies the size by initializing the TapeminiclassExtensionSize member in TAPE_INIT_DATA.  The TapeClassInitialize routine (in the Tapeclass driver) allocates the storage.  From there on, a pointer to this allocated memory is passed to the Tapeminiclass routines.

This storage is commenly used to maintain the inquery data associated with a device.   Thus if one Tapeminiclass driver supports many drives, then it can examine the TapeminiclassExtension data instead of querying the drive for the is Inquery data to determine which device it is operating on.

## Specifying DefaultTimeOutValue

The DefaultTimeOutValue is the number of seconds that the tape driver will wait when processing SRB requests before canceling them.  If a DefaultTimeOutValue of 0 is supplied then the Tapeclass driver  will choose an appropriate default value (currently this is set to 180).  READ and WRITE requests will use this default time out value.  ProcessCommand routines may override the default time out value in the SRB on a case by case basis.

## Specifying CommandExtensionSize

The tapeminiclass driver entrypoints receive a CallNumber parameter to determine the current state of the operation.   For some commands this is not enough.  They require the Tapeclass driver to maintain information in between calls.  This information can be stored in the memory pointed to by the CommandExtension parameter passed to the Tapeminiclass routine.  This memory is allocated by the Tapeclass driver at initialization time.   The amount of memory allocated is determined by the CommandExtensionSize member of the TAPE_INIT_DATA strucuture.

## VerifyInquiry routine

The Tapeminiclass’s VerifyInquiry routine is called by the Tapeclass driver at initialization time to determine whether or not the driver recognizes the tape drive.  The VerifyInquiry routine should return TRUE if the tape unit is supported by the Tapeminiclass driver or FALSE otherwise.

The VerifyInquiry routine is called with SCSI inquiry data and the mode capabilities page (mode page 2A as defined by the QIC 157 specification).  The latter argument is NULL if the tape drive doesn’t support the mode capabilities page.

The VerifyInquiry routine usually looks at the vendor and product id information contained in the inquiry data to determine if the tape unit is supported.  The TapeClassCompareMemory function is useful here as it provides an easy way to determine if two strings are equal.

# ProcessCommand routines

The ProcessCommand routines are used to implement the tape IOCTLs which in turn implement the Win32 tape API.  For NT, the inputs and outputs for these IOCTLs are defined in the NTDDTAPE.H include file contained in the Windows NT DDK.

The explanation for ProcessCommand routines is divided into two parts.  First a general discussion for implementing any ProcessCommand routine and then a description for each different ProcessCommand routine.

## General aspects of all ProcessCommand routines

ProcessCommand routines are called to process TAPE IOCTLs.  The parameters to a ProcessCommand routine are:

- MinitapeExtension - This supplies the per-device minitape extension.

- CommandExtension - This supplies the per-IOCTL command extension.

- CommandParameters	- This supplies the input/output buffer for the IOCTL as described in NTDDTAPE.H.

- Srb - This supplies a partially filled in SRB.  A subsequent section describes which fields in this structure should be modified by the ProcessCommand routine.

- CallNumber - This value is 0 the first time the ProcessCommand routine is called for an incoming TAPE IOCTL call.  The value is incremented by 1 for each subsequent call to the ProcessCommand routine until the IOCTL request is complete.

- LastError - This value is the error returned from the prevous SRB.

- RetryFlags - This value helps specify what action the tape driver should take when the tape device reports a failure.

The ProcessCommand routines return a TAPE_STATUS return code.  A return code of TAPE_STATUS_SEND_SRB_AND_CALLBACK indicates to the Tapeclass driver that the Srb parameter has been filled in and is ready to be executed by the tape device. By default, the minitape driver’s ProcessCommand routine will be called back if the SRB command is successfully completed by the tape drive.  This default behavior may be altered slightly by setting the RetryFlags parameter.

A return value of TAPE_CHECK_TAPE_UNIT_READY indicates to the Tapeclass driver that it should initialize a standard Test Unit Ready CDB and send it to the device.   If the CDB is successful, then the ProcessCommand routine is called back.

A return code other than TAPE_STATUS_SEND_SRB_AND_CALLBACK or TAPE_STATUS_CHECK_TAPE_UNIT_READY indicates to the Tapeclass driver that the IOCTL is complete.  The returned TAPE_STATUS code indicates success, failure, or warning as described in the section on TAPE_STATUS codes.

The SRB fields that a ProcessCommand routine must fill in before returning TAPE_STATUS_SEND_SRB_AND_CALLBACK are:

- CdbLength - Supplies the number of bytes in the SCSI CDB.

- Cdb - The SCSI CDB command.

- DataTransferLength - The number of bytes to be transferred for this SCSI command.  This value may not need to be explicitly set if ScsiTapeAllocateSrbBuffer is used.

The SRB fields that a ProcessCommand routine may optionally fill in are:

- DataBuffer - By using ScsiTapeAllocateSrbBuffer, a ProcessCommand routine must allocate a DataBuffer of length greater than or equal to DataTransferLength.

- TimeOutValue - A ProcessCommand routine may override the default time out value by setting the desired time out value in this field.

- SrbFlags - A ProcessCommand routine must turn on the SRB_FLAGS_DATA_OUT bit if the SCSI command is sending data to the tape drive.  This field doesn’t need to be changed if a SCSI command is receiving data from the tape drive or when no data is being transferred by the command.

The ProcessCommand routine may also optionally fill in the RetryFlags parameter.  The low 16 bits of the RetryFlags parameter specify the number of retries that the tape driver should perform in the event of a SCSI command failure.  The IGNORE_ERRORS bit in RetryFlags, if set, indicates to the tape driver that the tape driver should return control to the ProcessCommand routine even if the SCSI command failed. The RETURN_ERRORS bit in RetryFlags, if set, indicates to the Tapeclass driver that it should set the LastError parameter and  return control to the ProcessCommand routine even if the SCSI command failed.  If this bit is not set, which is the usual case, then any failure by the tape driver in processing the SCSI command results in the IOCTL request being completed with that failure.

Three ProcessCommand code snippets follow for executing three different kind of SCSI commands:  one that transfers data from the tape device, one that transfers data to the tape device, and one in which no data transfer is involved.

### Example:  SCSI Mode Sense request

PCDB                    cdb = (PCDB) Srb->Cdb;

PMODE_MEDIUM_PART_PAGE  mediumPage;

.

.

.

if (CallNumber == 2) {

if (!TapeClassAllocateSrbBuffer(Srb, sizeof(MODE_MEDIUM_PART_PAGE))) {

return TAPE_STATUS_INSUFFICIENT_RESOURCES;

}

mediumPage = Srb->DataBuffer;

Srb->CdbLength = CDB6GENERIC_LENGTH;

cdb->MODE_SENSE.OperationCode = SCSIOP_MODE_SENSE;

cdb->MODE_SENSE.Dbd = 1;

cdb->MODE_SENSE.PageCode = MODE_PAGE_MEDIUM_PARTITION;

cdb->MODE_SENSE.Pc = 1;

cdb->MODE_SENSE.AllocationLength = sizeof(MODE_MEDIUM_PART_PAGE) - 4;

Srb->DataTransferLength = sizeof(MODE_MEDIUM_PART_PAGE) - 4;

return TAPE_STATUS_SEND_SRB_AND_CALLBACK;

}

if (CallNumber == 3) {

mediumPage = Srb->DataBuffer;

if (!mediumPage->MediumPartPage.FDPBit) {

return TAPE_STATUS_INVALID_DEVICE_REQUEST;

}

.

.

.

### Example:  SCSI Mode Select request

PTAPE_CREATE_PARTITION  tapePartition = CommandParameters;

PCDB                    cdb = (PCDB) Srb->Cdb;

PMODE_MEDIUM_PART_PAGE  mediumPage

.

.

.

if (CallNumber == 4) {

mediumPage = Srb->DataBuffer;

if (tapePartition->Count == 0) {

mediumPage->MediumPartPage.FDPBit = 0;

} else if (tapePartition->Count == 2) {

mediumPage->MediumPartPage.FDPBit = 1;

} else {

ASSERT(FALSE);

}

// Set partitioning via the medium partition page.

Srb->CdbLength = CDB6GENERIC_LENGTH;

cdb->MODE_SELECT.OperationCode = SCSIOP_MODE_SELECT;

cdb->MODE_SELECT.PFBit = 1;

cdb->MODE_SELECT.ParameterListLength = sizeof(MODE_MEDIUM_PART_PAGE) - 4

Srb->DataTransferLength = sizeof(MODE_MEDIUM_PART_PAGE) - 4;

Srb->SrbFlags |= SRB_FLAGS_DATA_OUT;

return TAPE_STATUS_SEND_SRB_AND_CALLBACK;

}

.

.

.

### Example:  SCSI Rewind request

PCDB                    cdb = (PCDB) Srb->Cdb;

.

.

.

if (CallNumber == 1) {

Srb->CdbLength = CDB6GENERIC_LENGTH;

cdb->CDB6GENERIC.OperationCode = SCSIOP_REWIND;

Srb->DataTransferLength = 0;

return TAPE_STATUS_SEND_SRB_AND_CALLBACK;

}

.

.

.

## Descriptions for specific ProcessCommand routines

The goal of every ProcessCommand routine is to implement the associated command and return any requested information about the tape drive or tape media.  The following table provides lists all of the ProcessCommand routines, the corresponding NT IOCTL name, and the associated Win32 tape API.  The semantics of these IOCTLs are described in the documentation for the associated Win32 tape API.  The structure for the CommandParameters structures are described in NTDDTAPE.H.

| CreatePartition | IOCTL_TAPE_CREATE_PARTITION | CreateTapePartition |
| --- | --- | --- |
| Erase | IOCTL_TAPE_ERASE | EraseTape |
| GetDriveParameters | IOCTL_TAPE_GET_DRIVE_PARAMS | GetTapeParameters |
| GetMediaParameters | IOCTL_TAPE_GET_MEDIA_PARAMS | GetTapeParameters |
| GetPosition | IOCTL_TAPE_GET_POSITION | GetTapePosition |
| GetStatus | IOCTL_TAPE_GET_STATUS | GetTapeStatus |
| Prepare | IOCTL_TAPE_PREPARE | PrepareTape |
| SetDriveParameters | IOCTL_TAPE_SET_DRIVE_PARAMS | SetTapeParameters |
| SetMediaParameters | IOCTL_TAPE_SET_MEDIA_PARAMS | SetTapeParameters |
| SetPosition | IOCTL_TAPE_SET_POSITION | SetTapePosition |
| WriteMarks | IOCTL_TAPE_WRITE_MARKS | WriteTapemark |

# TapeError routine

A tapeminiclass driver may optionally specify a TapeError routine when calling ScsiTapeInitialize.  When a tape device error occurs when processing a READ, WRITE, or IOCTL, the tapeclass examines information returned by the tape drive to determine the cause of the failure and determine an appropriate TAPE_STATUS code.  By supplying a TapeError routine, the tapeminiclass driver gets a chance to second guess the tape driver.  The TapeError routine will be called with the Tapeminiclass extension, the failing SRB, and the TAPE_STATUS code that would be returned.

If the SRB_STATUS_AUTOSENSE_VALID bit is set in the SrbStatus field of the SRB then the SRB contains the SCSI sense information in the SenseInfoBuffer field with its length in the SenseInfoBufferLength.  The TapeError routine should examine this information along with the SrbStatus and ScsiStatus fields in the SRB in order to decide whether or not to change the TAPE_STATUS returned.

For most Tapeminiclass drivers, this function should not be necessary.

# TAPE_STATUS codes

Here is a list of all of the TAPE_STATUS codes, which Windows status codes that they map to and whether they indicate success, failure, or warning.  In the case of TAPE IOCTLs, a warning and an error both indicate that the requested operation did not complete.  For READ and WRITE requests, an error indicates failure and no data transferred while a warning indicates that the operation was terminated prematurely and so the application needs to check the bytes transferred returned to see how many bytes were read or written during the operation.  The Win32 TAPE API reference contains explanations for all of the Win32 error codes below.

TAPE_STATUS_SEND_SRB_AND_CALLBACK is not listed below because it has special meaning and will never be the final return code.

| TAPE_STATUS_SUCCESS | NO_ERROR | Success |
| --- | --- | --- |
| TAPE_STATUS_INSUFFICIENT_RESOURCES | ERROR_NOT_ENOUGH_MEMORY | Error |
| TAPE_STATUS_NOT_IMPLEMENTED | ERROR_INVALID_FUNCTION | Error |
| TAPE_STATUS_INVALID_DEVICE_REQUEST | ERROR_INVALID_FUNCTION | Error |
| TAPE_STATUS_INVALID_PARAMETER | ERROR_INVALID_PARAMETER | Error |
| TAPE_STATUS_MEDIA_CHANGED | ERROR_MEDIA_CHANGED | Warning |
| TAPE_STATUS_BUS_RESET | ERROR_BUS_RESET | Warning |
| TAPE_STATUS_SETMARK_DETECTED | ERROR_SETMARK_DETECTED | Warning |
| TAPE_STATUS_FILEMARK_DETECTED | ERROR_FILEMARK_DETECTED | Warning |
| TAPE_STATUS_BEGINNING_OF_MEDIA | ERROR_BEGINNING_OF_MEDIA | Warning |
| TAPE_STATUS_END_OF_MEDIA | ERROR_END_OF_MEDIA | Warning |
| TAPE_STATUS_BUFFER_OVERFLOW | ERROR_MORE_DATA | Warning |
| TAPE_STATUS_NO_DATA_DETECTED | ERROR_NO_DATA_DETECTED | Warning |
| TAPE_STATUS_EOM_OVERFLOW | ERROR_EOM_OVERFLOW | Error |
| TAPE_STATUS_NO_MEDIA | ERROR_NO_MEDIA_IN_DRIVE | Error |
| TAPE_STATUS_IO_DEVICE_ERROR | ERROR_IO_DEVICE | Error |
| TAPE_STATUS_UNRECOGNIZED_MEDIA | ERROR_UNRECOGNIZED_MEDIA | Error |
| TAPE_STATUS_DEVICE_NOT_READY | ERROR_NOT_READY | Error |
| TAPE_STATUS_MEDIA_WRITE_PROTECTED | ERROR_WRITE_PROTECT | Error |
| TAPE_STATUS_DEVICE_DATA_ERROR | ERROR_CRC | Error |
| TAPE_STATUS_NO_SUCH_DEVICE | ERROR_FILE_NOT_FOUND | Error |
| TAPE_STATUS_INVALID_BLOCK_LENGTH | ERROR_INVALID_BLOCK_LENGTH | Error |
| TAPE_STATUS_IO_TIMEOUT | ERROR_SEM_TIMEOUT | Error |
| TAPE_STATUS_DEVICE_NOT_CONNECTED | ERROR_NOT_READY | Error |
| TAPE_STATUS_DATA_OVERRUN | ERROR_IO_DEVICE | Error |
| TAPE_STATUS_DEVICE_BUSY | ERROR_BUSY | Warning |

# Tape/Minitape interface definitions

Following is an excerpt from the MINITAPE.H include file which defines the interface between the tape and minitape drivers.  It includes the definitions for all of the ScsiTapeXxx routines.

typedef

BOOLEAN

(*TAPE_VERIFY_INQUIRY_ROUTINE)(

IN  PINQUIRYDATA            InquiryData,

IN  	PMODE_CAPABILITIES_PAGE ModeCapabilitiesPage

);

typedef

VOID

(*EXTENSION_INIT_ROUTINE)(

IN  PVOID                   MinitapeExtension,

IN  PINQUIRYDATA            InquiryData,

IN  PMODE_CAPABILITIES_PAGE ModeCapabilitiesPage

);

typedef enum _TAPE_STATUS {

TAPE_STATUS_SEND_SRB_AND_CALLBACK,

TAPE_STATUS_CHECJ_TEST_UNIT_READY,

TAPE_STATUS_SUCCESS,

TAPE_STATUS_INSUFFICIENT_RESOURCES,

TAPE_STATUS_NOT_IMPLEMENTED,

TAPE_STATUS_INVALID_DEVICE_REQUEST,

TAPE_STATUS_INVALID_PARAMETER,

TAPE_STATUS_MEDIA_CHANGED,

TAPE_STATUS_BUS_RESET,

TAPE_STATUS_SETMARK_DETECTED,

TAPE_STATUS_FILEMARK_DETECTED,

TAPE_STATUS_BEGINNING_OF_MEDIA,

TAPE_STATUS_END_OF_MEDIA,

TAPE_STATUS_BUFFER_OVERFLOW,

TAPE_STATUS_NO_DATA_DETECTED,

TAPE_STATUS_EOM_OVERFLOW,

TAPE_STATUS_NO_MEDIA,

TAPE_STATUS_IO_DEVICE_ERROR,

TAPE_STATUS_UNRECOGNIZED_MEDIA,

TAPE_STATUS_DEVICE_NOT_READY,

TAPE_STATUS_MEDIA_WRITE_PROTECTED,

TAPE_STATUS_DEVICE_DATA_ERROR,

TAPE_STATUS_NO_SUCH_DEVICE,

TAPE_STATUS_INVALID_BLOCK_LENGTH,

TAPE_STATUS_IO_TIMEOUT,

TAPE_STATUS_DEVICE_NOT_CONNECTED,

TAPE_STATUS_DATA_OVERRUN,

TAPE_STATUS_DEVICE_BUSY

} TAPE_STATUS, *PTAPE_STATUS;

typedef

VOID

(*TAPE_ERROR_ROUTINE)(

IN      PVOID               MinitapeExtension,

IN      PSCSI_REQUEST_BLOCK Srb,

IN OUT  PTAPE_STATUS        TapeStatus

);

#define TAPE_RETRY_MASK 0x0000FFFF

#define IGNORE_ERRORS   0x00010000

#define RETURN_ERRORS   0x00020000

typedef

TAPE_STATUS

(*TAPE_PROCESS_COMMAND_ROUTINE)(

IN OUT  PVOID               MinitapeExtension,

IN OUT  PVOID               CommandExtension,

IN OUT  PVOID               CommandParameters,

IN OUT  PSCSI_REQUEST_BLOCK Srb,

IN      ULONG               CallNumber,

IN      TAPE_STATUS         LastError,

IN OUT  PULONG              RetryFlags

);

typedef struct _TAPE_INIT_DATA {

TAPE_VERIFY_INQUIRY_ROUTINE     VerifyInquiry;

ULONG                           MinitapeExtensionSize;

EXTENSION_INIT_ROUTINE          ExtensionInit;          /* OPTIONAL */

ULONG                           DefaultTimeOutValue;    /* OPTIONAL */

TAPE_ERROR_ROUTINE              TapeError;              /* OPTIONAL */

ULONG                           CommandExtensionSize;

TAPE_PROCESS_COMMAND_ROUTINE    CreatePartition;

TAPE_PROCESS_COMMAND_ROUTINE    Erase;

TAPE_PROCESS_COMMAND_ROUTINE    GetDriveParameters;

TAPE_PROCESS_COMMAND_ROUTINE    GetMediaParameters;

TAPE_PROCESS_COMMAND_ROUTINE    GetPosition;

TAPE_PROCESS_COMMAND_ROUTINE    GetStatus;

TAPE_PROCESS_COMMAND_ROUTINE    Prepare;

TAPE_PROCESS_COMMAND_ROUTINE    SetDriveParameters;

TAPE_PROCESS_COMMAND_ROUTINE    SetMediaParameters;

TAPE_PROCESS_COMMAND_ROUTINE    SetPosition;

TAPE_PROCESS_COMMAND_ROUTINE    WriteMarks;

} TAPE_INIT_DATA, *PTAPE_INIT_DATA;

SCSIPORT_API

ULONG

TapeClassInitialize(

IN  PVOID           Argument1,

IN  PVOID           Argument2,

IN  PTAPE_INIT_DATA TapeInitData

);

SCSIPORT_API

BOOLEAN

TapeClassAllocateSrbBuffer(

IN OUT  PSCSI_REQUEST_BLOCK Srb,

IN      ULONG               SrbBufferSize

);

SCSIPORT_API

VOID

TapeClassZeroMemory(

IN OUT  PVOID   Buffer,

IN      ULONG   BufferSize

);

SCSIPORT_API

ULONG

TapeClassCompareMemory(

IN OUT  PVOID   Source1,

IN OUT  PVOID   Source2,

IN      ULONG   Length

);

# MINITAPE.H

/*++

Copyright (c000294  Microsoft Corporation

Module Name:

minitape.h

Abstract:

Type definitions for minitape drivers.

Author:

Norbert Kusters 20-December-1994

Revision History:

--*/

#ifndef _MINITAPE_

#define _MINITAPE_

#include "stddef.h"

#define ASSERT( exp )

#ifndef IN

#define IN

#endif

#ifndef OUT

#define OUT

#endif

#ifndef OPTIONAL

#define OPTIONAL

#endif

#ifndef NOTHING

#define NOTHING

#endif

#ifndef CRITICAL

#define CRITICAL

#endif

#ifndef ANYSIZE_ARRAY

#define ANYSIZE_ARRAY 1       // winnt

#endif

// begin_winnt

#if defined(_MIPS_) && !(defined(MIDL_PASS) || defined(RC_INVOKED)) && defined(ENABLE_RESTRICTED)

#define RESTRICTED_POINTER __restrict

#else

#define RESTRICTED_POINTER

#endif

#if defined(_MIPS_) || defined(_ALPHA_) || defined(_PPC_)

#define UNALIGNED __unaligned

#else

#define UNALIGNED

#endif

// end_winnt

#ifndef CONST

#define CONST               const

#endif

// begin_winnt

#if (defined(_M_MRX000) || defined(_M_IX86) || defined(_M_ALPHA) || defined(_M_PPC)) && !defined(MIDL_PASS)

#define DECLSPEC_IMPORT __declspec(dllimport)

#else

#define DECLSPEC_IMPORT

#endif

// end_winnt

//

// Void

//

typedef void *PVOID;    // winnt

//

// Basics

//

#ifndef VOID

#define VOID void

typedef char CHAR;

typedef short SHORT;

typedef long LONG;

#endif

//

// UNICODE (Wide Character) types

//

typedef wchar_t WCHAR;    // wc,   16-bit UNICODE character

typedef WCHAR *PWCHAR;

typedef WCHAR *LPWCH, *PWCH;

typedef CONST WCHAR *LPCWCH, *PCWCH;

typedef WCHAR *NWPSTR;

typedef WCHAR *LPWSTR, *PWSTR;

typedef CONST WCHAR *LPCWSTR, *PCWSTR;

//

// ANSI (Multi-byte Character) types

//

typedef CHAR *PCHAR;

typedef CHAR *LPCH, *PCH;

typedef CONST CHAR *LPCCH, *PCCH;

typedef CHAR *NPSTR;

typedef CHAR *LPSTR, *PSTR;

typedef CONST CHAR *LPCSTR, *PCSTR;

//

// Neutral ANSI/UNICODE types and macros

//

#ifdef  UNICODE

#ifndef _TCHAR_DEFINED

typedef WCHAR TCHAR, *PTCHAR;

typedef WCHAR TUCHAR, *PTUCHAR;

#define _TCHAR_DEFINED

#endif /* !_TCHAR_DEFINED */

typedef LPWSTR LPTCH, PTCH;

typedef LPWSTR PTSTR, LPTSTR;

typedef LPCWSTR LPCTSTR;

typedef LPWSTR LP;

#define __TEXT(quote) L##quote

#else   /* UNICODE */

#ifndef _TCHAR_DEFINED

typedef char TCHAR, *PTCHAR;

typedef unsigned char TUCHAR, *PTUCHAR;

#define _TCHAR_DEFINED

#endif /* !_TCHAR_DEFINED */

typedef LPSTR LPTCH, PTCH;

typedef LPSTR PTSTR, LPTSTR;

typedef LPCSTR LPCTSTR;

#define __TEXT(quote) quote

#endif /* UNICODE */

#define TEXT(quote) __TEXT(quote)

// end_winnt

typedef double DOUBLE;

typedef struct _QUAD {              // QUAD is for those times we want

double  DoNotUseThisField;      // an 8 byte aligned 8 byte long structure

} QUAD;                             // which is NOT really a floating point

// number.  Use DOUBLE if you want an FP

// number.

//

// Pointer to Basics

//

typedef SHORT *PSHORT;  // winnt

typedef LONG *PLONG;    // winnt

typedef QUAD *PQUAD;

//

// Unsigned Basics

//

// Tell windef.h that some types are already defined.

#define BASETYPES

typedef unsigned char UCHAR;

typedef unsigned short USHORT;

typedef unsigned long ULONG;

typedef QUAD UQUAD;

//

// Pointer to Unsigned Basics

//

typedef UCHAR *PUCHAR;

typedef USHORT *PUSHORT;

typedef ULONG *PULONG;

typedef UQUAD *PUQUAD;

//

// Signed characters

//

typedef signed char SCHAR;

typedef SCHAR *PSCHAR;

#ifndef NO_STRICT

#ifndef STRICT

#define STRICT 1

#endif

#endif

//

// Handle to an Object

//

// begin_winnt

#ifdef STRICT

typedef void *HANDLE;

#define DECLARE_HANDLE(name) struct name##__ { int unused; }; typedef struct name##__ *name

#else

typedef PVOID HANDLE;

#define DECLARE_HANDLE(name) typedef HANDLE name

#endif

typedef HANDLE *PHANDLE;

// end_winnt

//

// Low order two bits of a handle are ignored by the system and available

// for use by application code as tag bits.  The remaining bits are opaque

// and used to store a serial number and table index.

//

#define OBJ_HANDLE_TAGBITS  0x00000003L

//

// Cardinal Data Types [0 - 2**N-2)

//

typedef char CCHAR;          // winnt

typedef short CSHORT;

typedef ULONG CLONG;

typedef CCHAR *PCCHAR;

typedef CSHORT *PCSHORT;

typedef CLONG *PCLONG;

//

// __int64 is only supported by 2.0 and later midl.

// __midl is set by the 2.0 midl and not by 1.0 midl.

//

#if (!defined(MIDL_PASS) || defined(__midl)) && (!defined(_M_IX86) || (defined(_INTEGRAL_MAX_BITS) && _INTEGRAL_MAX_BITS >= 64))

typedef __int64 LONGLONG;

typedef unsigned __int64 ULONGLONG;

#else

typedef double LONGLONG;

typedef double ULONGLONG;

#endif

typedef LONGLONG *PLONGLONG;

typedef ULONGLONG *PULONGLONG;

// Update Sequence Number

typedef LONGLONG USN;

#if defined(MIDL_PASS)

typedef struct _LARGE_INTEGER {

#else // MIDL_PASS

typedef union _LARGE_INTEGER {

struct {

ULONG LowPart;

LONG HighPart;

};

struct {

ULONG LowPart;

LONG HighPart;

} u;

#endif //MIDL_PASS

LONGLONG QuadPart;

} LARGE_INTEGER;

typedef LARGE_INTEGER *PLARGE_INTEGER;

#if defined(MIDL_PASS)

typedef struct _ULARGE_INTEGER {

#else // MIDL_PASS

typedef union _ULARGE_INTEGER {

struct {

ULONG LowPart;

ULONG HighPart;

};

struct {

ULONG LowPart;

ULONG HighPart;

} u;

#endif //MIDL_PASS

ULONGLONG QuadPart;

} ULARGE_INTEGER;

typedef ULARGE_INTEGER *PULARGE_INTEGER;

//

// Boolean

//

typedef UCHAR BOOLEAN;           // winnt

typedef BOOLEAN *PBOOLEAN;       // winnt

//

// Constants

//

#define FALSE   0

#define TRUE    1

#ifndef NULL

#ifdef __cplusplus

#define NULL    0

#else

#define NULL    ((void *)0)

#endif

#endif // NULL

//

// Macros used to eliminate compiler warning generated when formal

// parameters or local variables are not declared.

//

// Use DBG_UNREFERENCED_PARAMETER() when a parameter is not yet

// referenced but will be once the module is completely developed.

//

// Use DBG_UNREFERENCED_LOCAL_VARIABLE() when a local variable is not yet

// referenced but will be once the module is completely developed.

//

// Use UNREFERENCED_PARAMETER() if a parameter will never be referenced.

//

// DBG_UNREFERENCED_PARAMETER and DBG_UNREFERENCED_LOCAL_VARIABLE will

// eventually be made into a null macro to help determine whether there

// is unfinished work.

//

#if ! (defined(lint) || defined(_lint))

#define UNREFERENCED_PARAMETER(P)          (P)

#define DBG_UNREFERENCED_PARAMETER(P)      (P)

#define DBG_UNREFERENCED_LOCAL_VARIABLE(V) (V)

#else // lint or _lint

// Note: lint -e530 says don't complain about uninitialized variables for

// this.  line +e530 turns that checking back on.  Error 527 has to do with

// unreachable code.

#define UNREFERENCED_PARAMETER(P)          \

/*lint -e527 -e530 */ \

{ \

(P) = (P); \

} \

/*lint +e527 +e530 */

#define DBG_UNREFERENCED_PARAMETER(P)      \

/*lint -e527 -e530 */ \

{ \

(P) = (P); \

} \

/*lint +e527 +e530 */

#define DBG_UNREFERENCED_LOCAL_VARIABLE(V) \

/*lint -e527 -e530 */ \

{ \

(V) = (V); \

} \

/*lint +e527 +e530 */

#endif // lint or _lint

//

// IOCTL_TAPE_ERASE definitions

//

#define TAPE_ERASE_SHORT            0L

#define TAPE_ERASE_LONG             1L

typedef struct _TAPE_ERASE {

ULONG Type;

BOOLEAN Immediate;

} TAPE_ERASE, *PTAPE_ERASE;

//

// IOCTL_TAPE_PREPARE definitions

//

#define TAPE_LOAD                   0L

#define TAPE_UNLOAD                 1L

#define TAPE_TENSION                2L

#define TAPE_LOCK                   3L

#define TAPE_UNLOCK                 4L

#define TAPE_FORMAT                 5L

typedef struct _TAPE_PREPARE {

ULONG Operation;

BOOLEAN Immediate;

} TAPE_PREPARE, *PTAPE_PREPARE;

//

// IOCTL_TAPE_WRITE_MARKS definitions

//

#define TAPE_SETMARKS               0L

#define TAPE_FILEMARKS              1L

#define TAPE_SHORT_FILEMARKS        2L

#define TAPE_LONG_FILEMARKS         3L

typedef struct _TAPE_WRITE_MARKS {

ULONG Type;

ULONG Count;

BOOLEAN Immediate;

} TAPE_WRITE_MARKS, *PTAPE_WRITE_MARKS;

//

// IOCTL_TAPE_GET_POSITION definitions

//

#define TAPE_ABSOLUTE_POSITION       0L

#define TAPE_LOGICAL_POSITION        1L

#define TAPE_PSEUDO_LOGICAL_POSITION 2L

typedef struct _TAPE_GET_POSITION {

ULONG Type;

ULONG Partition;

LARGE_INTEGER Offset;

} TAPE_GET_POSITION, *PTAPE_GET_POSITION;

//

// IOCTL_TAPE_SET_POSITION definitions

//

#define TAPE_REWIND                 0L

#define TAPE_ABSOLUTE_BLOCK         1L

#define TAPE_LOGICAL_BLOCK          2L

#define TAPE_PSEUDO_LOGICAL_BLOCK   3L

#define TAPE_SPACE_END_OF_DATA      4L

#define TAPE_SPACE_RELATIVE_BLOCKS  5L

#define TAPE_SPACE_FILEMARKS        6L

#define TAPE_SPACE_SEQUENTIAL_FMKS  7L

#define TAPE_SPACE_SETMARKS         8L

#define TAPE_SPACE_SEQUENTIAL_SMKS  9L

typedef struct _TAPE_SET_POSITION {

ULONG Method;

ULONG Partition;

LARGE_INTEGER Offset;

BOOLEAN Immediate;

} TAPE_SET_POSITION, *PTAPE_SET_POSITION;

//

// IOCTL_TAPE_GET_DRIVE_PARAMS definitions

//

//

// Definitions for FeaturesLow parameter

//

#define TAPE_DRIVE_FIXED            0x00000001

#define TAPE_DRIVE_SELECT           0x00000002

#define TAPE_DRIVE_INITIATOR        0x00000004

#define TAPE_DRIVE_ERASE_SHORT      0x00000010

#define TAPE_DRIVE_ERASE_LONG       0x00000020

#define TAPE_DRIVE_ERASE_BOP_ONLY   0x00000040

#define TAPE_DRIVE_ERASE_IMMEDIATE  0x00000080

#define TAPE_DRIVE_TAPE_CAPACITY    0x00000100

#define TAPE_DRIVE_TAPE_REMAINING   0x00000200

#define TAPE_DRIVE_FIXED_BLOCK      0x00000400

#define TAPE_DRIVE_VARIABLE_BLOCK   0x00000800

#define TAPE_DRIVE_WRITE_PROTECT    0x00001000

#define TAPE_DRIVE_EOT_WZ_SIZE      0x00002000

#define TAPE_DRIVE_ECC              0x00010000

#define TAPE_DRIVE_COMPRESSION      0x00020000

#define TAPE_DRIVE_PADDING          0x00040000

#define TAPE_DRIVE_REPORT_SMKS      0x00080000

#define TAPE_DRIVE_GET_ABSOLUTE_BLK 0x00100000

#define TAPE_DRIVE_GET_LOGICAL_BLK  0x00200000

#define TAPE_DRIVE_SET_EOT_WZ_SIZE  0x00400000

#define TAPE_DRIVE_RESERVED_BIT     0x80000000  //don't use this bit!

//                                              //can't be a low features bit!

//                                              //reserved; high features only

//

// Definitions for FeaturesHigh parameter

//

#define TAPE_DRIVE_LOAD_UNLOAD      0x80000001

#define TAPE_DRIVE_TENSION          0x80000002

#define TAPE_DRIVE_LOCK_UNLOCK      0x80000004

#define TAPE_DRIVE_REWIND_IMMEDIATE 0x80000008

#define TAPE_DRIVE_SET_BLOCK_SIZE   0x80000010

#define TAPE_DRIVE_LOAD_UNLD_IMMED  0x80000020

#define TAPE_DRIVE_TENSION_IMMED    0x80000040

#define TAPE_DRIVE_LOCK_UNLK_IMMED  0x80000080

#define TAPE_DRIVE_SET_ECC          0x80000100

#define TAPE_DRIVE_SET_COMPRESSION  0x80000200

#define TAPE_DRIVE_SET_PADDING      0x80000400

#define TAPE_DRIVE_SET_REPORT_SMKS  0x80000800

#define TAPE_DRIVE_ABSOLUTE_BLK     0x80001000

#define TAPE_DRIVE_ABS_BLK_IMMED    0x80002000

#define TAPE_DRIVE_LOGICAL_BLK      0x80004000

#define TAPE_DRIVE_LOG_BLK_IMMED    0x80008000

#define TAPE_DRIVE_END_OF_DATA      0x80010000

#define TAPE_DRIVE_RELATIVE_BLKS    0x80020000

#define TAPE_DRIVE_FILEMARKS        0x80040000

#define TAPE_DRIVE_SEQUENTIAL_FMKS  0x80080000

#define TAPE_DRIVE_SETMARKS         0x80100000

#define TAPE_DRIVE_SEQUENTIAL_SMKS  0x80200000

#define TAPE_DRIVE_REVERSE_POSITION 0x80400000

#define TAPE_DRIVE_SPACE_IMMEDIATE  0x80800000

#define TAPE_DRIVE_WRITE_SETMARKS   0x81000000

#define TAPE_DRIVE_WRITE_FILEMARKS  0x82000000

#define TAPE_DRIVE_WRITE_SHORT_FMKS 0x84000000

#define TAPE_DRIVE_WRITE_LONG_FMKS  0x88000000

#define TAPE_DRIVE_WRITE_MARK_IMMED 0x90000000

#define TAPE_DRIVE_FORMAT           0xA0000000

#define TAPE_DRIVE_FORMAT_IMMEDIATE 0xC0000000

#define TAPE_DRIVE_HIGH_FEATURES    0x80000000  //mask for high features flag

typedef struct _TAPE_GET_DRIVE_PARAMETERS {

BOOLEAN ECC;

BOOLEAN Compression;

BOOLEAN DataPadding;

BOOLEAN ReportSetmarks;

ULONG DefaultBlockSize;

ULONG MaximumBlockSize;

ULONG MinimumBlockSize;

ULONG MaximumPartitionCount;

ULONG FeaturesLow;

ULONG FeaturesHigh;

ULONG EOTWarningZoneSize;

} TAPE_GET_DRIVE_PARAMETERS, *PTAPE_GET_DRIVE_PARAMETERS;

//

// IOCTL_TAPE_SET_DRIVE_PARAMETERS definitions

//

typedef struct _TAPE_SET_DRIVE_PARAMETERS {

BOOLEAN ECC;

BOOLEAN Compression;

BOOLEAN DataPadding;

BOOLEAN ReportSetmarks;

ULONG EOTWarningZoneSize;

} TAPE_SET_DRIVE_PARAMETERS, *PTAPE_SET_DRIVE_PARAMETERS;

//

// IOCTL_TAPE_GET_MEDIA_PARAMETERS definitions

//

typedef struct _TAPE_GET_MEDIA_PARAMETERS {

LARGE_INTEGER Capacity;

LARGE_INTEGER Remaining;

ULONG BlockSize;

ULONG PartitionCount;

BOOLEAN WriteProtected;

} TAPE_GET_MEDIA_PARAMETERS, *PTAPE_GET_MEDIA_PARAMETERS;

//

// IOCTL_TAPE_SET_MEDIA_PARAMETERS definitions

//

typedef struct _TAPE_SET_MEDIA_PARAMETERS {

ULONG BlockSize;

} TAPE_SET_MEDIA_PARAMETERS, *PTAPE_SET_MEDIA_PARAMETERS;

//

// IOCTL_TAPE_CREATE_PARTITION definitions

//

#define TAPE_FIXED_PARTITIONS       0L

#define TAPE_SELECT_PARTITIONS      1L

#define TAPE_INITIATOR_PARTITIONS   2L

typedef struct _TAPE_CREATE_PARTITION {

ULONG Method;

ULONG Count;

ULONG Size;

} TAPE_CREATE_PARTITION, *PTAPE_CREATE_PARTITION;

#define MAXIMUM_CDB_SIZE 12

//

// SCSI I/O Request Block

//

typedef struct _SCSI_REQUEST_BLOCK {

USHORT Length;                  // offset 0

UCHAR Function;                 // offset 2

UCHAR SrbStatus;                // offset 3

UCHAR ScsiStatus;               // offset 4

UCHAR PathId;                   // offset 5

UCHAR TargetId;                 // offset 6

UCHAR Lun;                      // offset 7

UCHAR QueueTag;                 // offset 8

UCHAR QueueAction;              // offset 9

UCHAR CdbLength;                // offset a

UCHAR SenseInfoBufferLength;    // offset b

ULONG SrbFlags;                 // offset c

ULONG DataTransferLength;       // offset 10

ULONG TimeOutValue;             // offset 14

PVOID DataBuffer;               // offset 18

PVOID SenseInfoBuffer;          // offset 1c

struct _SCSI_REQUEST_BLOCK *NextSrb; // offset 20

PVOID OriginalRequest;          // offset 24

PVOID SrbExtension;             // offset 28

ULONG QueueSortKey;             // offset 2c

UCHAR Cdb[16];                  // offset 30

} SCSI_REQUEST_BLOCK, *PSCSI_REQUEST_BLOCK;

#define SCSI_REQUEST_BLOCK_SIZE sizeof(SCSI_REQUEST_BLOCK)

//

// SRB Functions

//

#define SRB_FUNCTION_EXECUTE_SCSI           0x00

#define SRB_FUNCTION_CLAIM_DEVICE           0x01

#define SRB_FUNCTION_IO_CONTROL             0x02

#define SRB_FUNCTION_RECEIVE_EVENT          0x03

#define SRB_FUNCTION_RELEASE_QUEUE          0x04

#define SRB_FUNCTION_ATTACH_DEVICE          0x05

#define SRB_FUNCTION_RELEASE_DEVICE         0x06

#define SRB_FUNCTION_SHUTDOWN               0x07

#define SRB_FUNCTION_FLUSH                  0x08

#define SRB_FUNCTION_ABORT_COMMAND          0x10

#define SRB_FUNCTION_RELEASE_RECOVERY       0x11

#define SRB_FUNCTION_RESET_BUS              0x12

#define SRB_FUNCTION_RESET_DEVICE           0x13

#define SRB_FUNCTION_TERMINATE_IO           0x14

#define SRB_FUNCTION_FLUSH_QUEUE            0x15

#define SRB_FUNCTION_REMOVE_DEVICE          0x16

//

// SRB Status

//

#define SRB_STATUS_PENDING                  0x00

#define SRB_STATUS_SUCCESS                  0x01

#define SRB_STATUS_ABORTED                  0x02

#define SRB_STATUS_ABORT_FAILED             0x03

#define SRB_STATUS_ERROR                    0x04

#define SRB_STATUS_BUSY                     0x05

#define SRB_STATUS_INVALID_REQUEST          0x06

#define SRB_STATUS_INVALID_PATH_ID          0x07

#define SRB_STATUS_NO_DEVICE                0x08

#define SRB_STATUS_TIMEOUT                  0x09

#define SRB_STATUS_SELECTION_TIMEOUT        0x0A

#define SRB_STATUS_COMMAND_TIMEOUT          0x0B

#define SRB_STATUS_MESSAGE_REJECTED         0x0D

#define SRB_STATUS_BUS_RESET                0x0E

#define SRB_STATUS_PARITY_ERROR             0x0F

#define SRB_STATUS_REQUEST_SENSE_FAILED     0x10

#define SRB_STATUS_NO_HBA                   0x11

#define SRB_STATUS_DATA_OVERRUN             0x12

#define SRB_STATUS_UNEXPECTED_BUS_FREE      0x13

#define SRB_STATUS_PHASE_SEQUENCE_FAILURE   0x14

#define SRB_STATUS_BAD_SRB_BLOCK_LENGTH     0x15

#define SRB_STATUS_REQUEST_FLUSHED          0x16

#define SRB_STATUS_INVALID_LUN              0x20

#define SRB_STATUS_INVALID_TARGET_ID        0x21

#define SRB_STATUS_BAD_FUNCTION             0x22

#define SRB_STATUS_ERROR_RECOVERY           0x23

//

// SRB Status Masks

//

#define SRB_STATUS_QUEUE_FROZEN             0x40

#define SRB_STATUS_AUTOSENSE_VALID          0x80

#define SRB_STATUS(Status) (Status & ~(SRB_STATUS_AUTOSENSE_VALID | SRB_STATUS_QUEUE_FROZEN))

//

// SRB Flag Bits

//

#define SRB_FLAGS_QUEUE_ACTION_ENABLE       0x00000002

#define SRB_FLAGS_DISABLE_DISCONNECT        0x00000004

#define SRB_FLAGS_DISABLE_SYNCH_TRANSFER    0x00000008

#define SRB_FLAGS_BYPASS_FROZEN_QUEUE       0x00000010

#define SRB_FLAGS_DISABLE_AUTOSENSE         0x00000020

#define SRB_FLAGS_DATA_IN                   0x00000040

#define SRB_FLAGS_DATA_OUT                  0x00000080

#define SRB_FLAGS_NO_DATA_TRANSFER          0x00000000

#define SRB_FLAGS_UNSPECIFIED_DIRECTION      (SRB_FLAGS_DATA_IN | SRB_FLAGS_DATA_OUT)

#define SRB_FLAGS_NO_QUEUE_FREEZE           0x00000100

#define SRB_FLAGS_ADAPTER_CACHE_ENABLE      0x00000200

#define SRB_FLAGS_IS_ACTIVE                 0x00010000

#define SRB_FLAGS_ALLOCATED_FROM_ZONE       0x00020000

#define SRB_FLAGS_SGLIST_FROM_POOL          0x00040000

//

// Queue Action

//

#define SRB_SIMPLE_TAG_REQUEST              0x20

#define SRB_HEAD_OF_QUEUE_TAG_REQUEST       0x21

#define SRB_ORDERED_QUEUE_TAG_REQUEST       0x22

#ifndef _NTDDK_

#define SCSIPORT_API DECLSPEC_IMPORT

#else

#define SCSIPORT_API

#endif

SCSIPORT_API

VOID

ScsiDebugPrint(

ULONG DebugPrintLevel,

PCCHAR DebugMessage,

...

);

//

// Command Descriptor Block. Passed by SCSI controller chip over the SCSI bus

//

typedef union _CDB {

//

// Generic 6-Byte CDB

//

struct _CDB6GENERIC {

UCHAR  OperationCode;

UCHAR  Immediate : 1;

UCHAR  CommandUniqueBits : 4;

UCHAR  LogicalUnitNumber : 3;

UCHAR  CommandUniqueBytes[3];

UCHAR  Link : 1;

UCHAR  Flag : 1;

UCHAR  Reserved : 4;

UCHAR  VendorUnique : 2;

} CDB6GENERIC, *PCDB6GENERIC;

//

// Standard 6-byte CDB

//

struct _CDB6READWRITE {

UCHAR OperationCode;

UCHAR LogicalBlockMsb1 : 5;

UCHAR LogicalUnitNumber : 3;

UCHAR LogicalBlockMsb0;

UCHAR LogicalBlockLsb;

UCHAR TransferBlocks;

UCHAR Control;

} CDB6READWRITE, *PCDB6READWRITE;

//

// SCSI Inquiry CDB

//

struct _CDB6INQUIRY {

UCHAR OperationCode;

UCHAR Reserved1 : 5;

UCHAR LogicalUnitNumber : 3;

UCHAR PageCode;

UCHAR IReserved;

UCHAR AllocationLength;

UCHAR Control;

} CDB6INQUIRY, *PCDB6INQUIRY;

//

// SCSI Format CDB

//

struct _CDB6FORMAT {

UCHAR OperationCode;

UCHAR FormatControl : 5;

UCHAR LogicalUnitNumber : 3;

UCHAR FReserved1;

UCHAR InterleaveMsb;

UCHAR InterleaveLsb;

UCHAR FReserved2;

} CDB6FORMAT, *PCDB6FORMAT;

//

// Standard 10-byte CDB

struct _CDB10 {

UCHAR OperationCode;

UCHAR RelativeAddress : 1;

UCHAR Reserved1 : 2;

UCHAR ForceUnitAccess : 1;

UCHAR DisablePageOut : 1;

UCHAR LogicalUnitNumber : 3;

UCHAR LogicalBlockByte0;

UCHAR LogicalBlockByte1;

UCHAR LogicalBlockByte2;

UCHAR LogicalBlockByte3;

UCHAR Reserved2;

UCHAR TransferBlocksMsb;

UCHAR TransferBlocksLsb;

UCHAR Control;

} CDB10, *PCDB10;

//

// CD Rom Audio CDBs

//

struct _PAUSE_RESUME {

UCHAR OperationCode;

UCHAR Reserved1 : 5;

UCHAR LogicalUnitNumber : 3;

UCHAR Reserved2[6];

UCHAR Action;

UCHAR Control;

} PAUSE_RESUME, *PPAUSE_RESUME;

//

// Read Table of Contents

//

struct _READ_TOC {

UCHAR OperationCode;

UCHAR Reserved0 : 1;

UCHAR Msf : 1;

UCHAR Reserved1 : 3;

UCHAR LogicalUnitNumber : 3;

UCHAR Reserved2[4];

UCHAR StartingTrack;

UCHAR AllocationLength[2];

UCHAR Control : 6;

UCHAR Format : 2;

} READ_TOC, *PREAD_TOC;

struct _PLAY_AUDIO_MSF {

UCHAR OperationCode;

UCHAR Reserved1 : 5;

UCHAR LogicalUnitNumber : 3;

UCHAR Reserved2;

UCHAR StartingM;

UCHAR StartingS;

UCHAR StartingF;

UCHAR EndingM;

UCHAR EndingS;

UCHAR EndingF;

UCHAR Control;

} PLAY_AUDIO_MSF, *PPLAY_AUDIO_MSF;

//

// Read SubChannel Data

//

struct _SUBCHANNEL {

UCHAR OperationCode;

UCHAR Reserved0 : 1;

UCHAR Msf : 1;

UCHAR Reserved1 : 3;

UCHAR LogicalUnitNumber : 3;

UCHAR Reserved2 : 6;

UCHAR SubQ : 1;

UCHAR Reserved3 : 1;

UCHAR Format;

UCHAR Reserved4[2];

UCHAR TrackNumber;

UCHAR AllocationLength[2];

UCHAR Control;

} SUBCHANNEL, *PSUBCHANNEL;

//

// Mode sense

//

struct _MODE_SENSE {

UCHAR OperationCode;

UCHAR Reserved1 : 3;

UCHAR Dbd : 1;

UCHAR Reserved2 : 1;

UCHAR LogicalUnitNumber : 3;

UCHAR PageCode : 6;

UCHAR Pc : 2;

UCHAR Reserved3;

UCHAR AllocationLength;

UCHAR Control;

} MODE_SENSE, *PMODE_SENSE;

//

// Mode select

//

struct _MODE_SELECT {

UCHAR OperationCode;

UCHAR SPBit : 1;

UCHAR Reserved1 : 3;

UCHAR PFBit : 1;

UCHAR LogicalUnitNumber : 3;

UCHAR Reserved2[2];

UCHAR ParameterListLength;

UCHAR Control;

} MODE_SELECT, *PMODE_SELECT;

struct _LOCATE {

UCHAR OperationCode;

UCHAR Immediate : 1;

UCHAR CPBit : 1;

UCHAR BTBit : 1;

UCHAR Reserved1 : 2;

UCHAR LogicalUnitNumber : 3;

UCHAR Reserved3;

UCHAR LogicalBlockAddress[4];

UCHAR Reserved4;

UCHAR Partition;

UCHAR Control;

} LOCATE, *PLOCATE;

struct _LOGSENSE {

UCHAR OperationCode;

UCHAR SPBit : 1;

UCHAR PPCBit : 1;

UCHAR Reserved1 : 3;

UCHAR LogicalUnitNumber : 3;

UCHAR PageCode : 6;

UCHAR PCBit : 2;

UCHAR Reserved2;

UCHAR Reserved3;

UCHAR ParameterPointer[2];  // [0]=MSB, [1]=LSB

UCHAR AllocationLength[2];  // [0]=MSB, [1]=LSB

UCHAR Control;

} LOGSENSE, *PLOGSENSE;

struct _PRINT {

UCHAR OperationCode;

UCHAR Reserved : 5;

UCHAR LogicalUnitNumber : 3;

UCHAR TransferLength[3];

UCHAR Control;

} PRINT, *PPRINT;

struct _SEEK {

UCHAR OperationCode;

UCHAR Reserved1 : 5;

UCHAR LogicalUnitNumber : 3;

UCHAR LogicalBlockAddress[4];

UCHAR Reserved2[3];

UCHAR Control;

} SEEK, *PSEEK;

struct _ERASE {

UCHAR OperationCode;

UCHAR Long : 1;

UCHAR Immediate : 1;

UCHAR Reserved1 : 3;

UCHAR LogicalUnitNumber : 3;

UCHAR Reserved2[3];

UCHAR Control;

} ERASE, *PERASE;

struct _START_STOP {

UCHAR OperationCode;

UCHAR Immediate: 1;

UCHAR Reserved1 : 4;

UCHAR LogicalUnitNumber : 3;

UCHAR Reserved2[2];

UCHAR Start : 1;

UCHAR LoadEject : 1;

UCHAR Reserved3 : 6;

UCHAR Control;

} START_STOP, *PSTART_STOP;

struct _MEDIA_REMOVAL {

UCHAR OperationCode;

UCHAR Reserved1 : 5;

UCHAR LogicalUnitNumber : 3;

UCHAR Reserved2[2];

UCHAR Prevent;

UCHAR Control;

} MEDIA_REMOVAL, *PMEDIA_REMOVAL;

//

// Tape CDBs

//

struct _SEEK_BLOCK {

UCHAR OperationCode;

UCHAR Immediate : 1;

UCHAR Reserved1 : 7;

UCHAR BlockAddress[3];

UCHAR Link : 1;

UCHAR Flag : 1;

UCHAR Reserved2 : 4;

UCHAR VendorUnique : 2;

} SEEK_BLOCK, *PSEEK_BLOCK;

struct _REQUEST_BLOCK_ADDRESS {

UCHAR OperationCode;

UCHAR Reserved1[3];

UCHAR AllocationLength;

UCHAR Link : 1;

UCHAR Flag : 1;

UCHAR Reserved2 : 4;

UCHAR VendorUnique : 2;

} REQUEST_BLOCK_ADDRESS, *PREQUEST_BLOCK_ADDRESS;

struct _PARTITION {

UCHAR OperationCode;

UCHAR Immediate : 1;

UCHAR Sel: 1;

UCHAR PartitionSelect : 6;

UCHAR Reserved1[3];

UCHAR Control;

} PARTITION, *PPARTITION;

struct _WRITE_TAPE_MARKS {

UCHAR OperationCode;

UCHAR Immediate : 1;

UCHAR WriteSetMarks: 1;

UCHAR Reserved : 3;

UCHAR LogicalUnitNumber : 3;

UCHAR TransferLength[3];

UCHAR Control;

} WRITE_TAPE_MARKS, *PWRITE_TAPE_MARKS;

struct _SPACE_TAPE_MARKS {

UCHAR OperationCode;

UCHAR Code : 3;

UCHAR Reserved : 2;

UCHAR LogicalUnitNumber : 3;

UCHAR NumMarksMSB ;

UCHAR NumMarks;

UCHAR NumMarksLSB;

union {

UCHAR value;

struct {

UCHAR Link : 1;

UCHAR Flag : 1;

UCHAR Reserved : 4;

UCHAR VendorUnique : 2;

} Fields;

} Byte6;

} SPACE_TAPE_MARKS, *PSPACE_TAPE_MARKS;

//

// Read tape position

//

struct _READ_POSITION {

UCHAR Operation;

UCHAR BlockType:1;

UCHAR Reserved1:4;

UCHAR Lun:3;

UCHAR Reserved2[7];

UCHAR Control;

} READ_POSITION, *PREAD_POSITION;

//

// ReadWrite for Tape

//

struct _CDB6READWRITETAPE {

UCHAR OperationCode;

UCHAR VendorSpecific : 5;

UCHAR Reserved : 3;

UCHAR TransferLenMSB;

UCHAR TransferLen;

UCHAR TransferLenLSB;

UCHAR Link : 1;

UCHAR Flag : 1;

UCHAR Reserved1 : 4;

UCHAR VendorUnique : 2;

} CDB6READWRITETAPE, *PCDB6READWRITETAPE;

} CDB, *PCDB;

//

// Command Descriptor Block constants.

//

#define CDB6GENERIC_LENGTH                   6

#define CDB10GENERIC_LENGTH                  10

#define SETBITON                             1

#define SETBITOFF                            0

//

// Mode Sense/Select page constants.

//

#define MODE_PAGE_ERROR_RECOVERY        0x01

#define MODE_PAGE_DISCONNECT            0x02

#define MODE_PAGE_FORMAT_DEVICE         0x03

#define MODE_PAGE_RIGID_GEOMETRY        0x04

#define MODE_PAGE_FLEXIBILE             0x05

#define MODE_PAGE_VERIFY_ERROR          0x07

#define MODE_PAGE_CACHING               0x08

#define MODE_PAGE_PERIPHERAL            0x09

#define MODE_PAGE_CONTROL               0x0A

#define MODE_PAGE_MEDIUM_TYPES          0x0B

#define MODE_PAGE_NOTCH_PARTITION       0x0C

#define MODE_SENSE_RETURN_ALL           0x3f

#define MODE_SENSE_CURRENT_VALUES       0x00

#define MODE_SENSE_CHANGEABLE_VALUES    0x40

#define MODE_SENSE_DEFAULT_VAULES       0x80

#define MODE_SENSE_SAVED_VALUES         0xc0

#define MODE_PAGE_DEVICE_CONFIG         0x10

#define MODE_PAGE_MEDIUM_PARTITION      0x11

#define MODE_PAGE_DATA_COMPRESS         0x0f

#define MODE_PAGE_CAPABILITIES          0x2A

//

// SCSI CDB operation codes

//

#define SCSIOP_TEST_UNIT_READY     0x00

#define SCSIOP_REZERO_UNIT         0x01

#define SCSIOP_REWIND              0x01

#define SCSIOP_REQUEST_BLOCK_ADDR  0x02

#define SCSIOP_REQUEST_SENSE       0x03

#define SCSIOP_FORMAT_UNIT         0x04

#define SCSIOP_READ_BLOCK_LIMITS   0x05

#define SCSIOP_REASSIGN_BLOCKS     0x07

#define SCSIOP_READ6               0x08

#define SCSIOP_RECEIVE             0x08

#define SCSIOP_WRITE6              0x0A

#define SCSIOP_PRINT               0x0A

#define SCSIOP_SEND                0x0A

#define SCSIOP_SEEK6               0x0B

#define SCSIOP_TRACK_SELECT        0x0B

#define SCSIOP_SLEW_PRINT          0x0B

#define SCSIOP_SEEK_BLOCK          0x0C

#define SCSIOP_PARTITION           0x0D

#define SCSIOP_READ_REVERSE        0x0F

#define SCSIOP_WRITE_FILEMARKS     0x10

#define SCSIOP_FLUSH_BUFFER        0x10

#define SCSIOP_SPACE               0x11

#define SCSIOP_INQUIRY             0x12

#define SCSIOP_VERIFY6             0x13

#define SCSIOP_RECOVER_BUF_DATA    0x14

#define SCSIOP_MODE_SELECT         0x15

#define SCSIOP_RESERVE_UNIT        0x16

#define SCSIOP_RELEASE_UNIT        0x17

#define SCSIOP_COPY                0x18

#define SCSIOP_ERASE               0x19

#define SCSIOP_MODE_SENSE          0x1A

#define SCSIOP_START_STOP_UNIT     0x1B

#define SCSIOP_STOP_PRINT          0x1B

#define SCSIOP_LOAD_UNLOAD         0x1B

#define SCSIOP_RECEIVE_DIAGNOSTIC  0x1C

#define SCSIOP_SEND_DIAGNOSTIC     0x1D

#define SCSIOP_MEDIUM_REMOVAL      0x1E

#define SCSIOP_READ_CAPACITY       0x25

#define SCSIOP_READ                0x28

#define SCSIOP_WRITE               0x2A

#define SCSIOP_SEEK                0x2B

#define SCSIOP_LOCATE              0x2B

#define SCSIOP_WRITE_VERIFY        0x2E

#define SCSIOP_VERIFY              0x2F

#define SCSIOP_SEARCH_DATA_HIGH    0x30

#define SCSIOP_SEARCH_DATA_EQUAL   0x31

#define SCSIOP_SEARCH_DATA_LOW     0x32

#define SCSIOP_SET_LIMITS          0x33

#define SCSIOP_READ_POSITION       0x34

#define SCSIOP_SYNCHRONIZE_CACHE   0x35

#define SCSIOP_COMPARE             0x39

#define SCSIOP_COPY_COMPARE        0x3A

#define SCSIOP_WRITE_DATA_BUFF     0x3B

#define SCSIOP_READ_DATA_BUFF      0x3C

#define SCSIOP_CHANGE_DEFINITION   0x40

#define SCSIOP_READ_SUB_CHANNEL    0x42

#define SCSIOP_READ_TOC            0x43

#define SCSIOP_READ_HEADER         0x44

#define SCSIOP_PLAY_AUDIO          0x45

#define SCSIOP_PLAY_AUDIO_MSF      0x47

#define SCSIOP_PLAY_TRACK_INDEX    0x48

#define SCSIOP_PLAY_TRACK_RELATIVE 0x49

#define SCSIOP_PAUSE_RESUME        0x4B

#define SCSIOP_LOG_SELECT          0x4C

#define SCSIOP_LOG_SENSE           0x4D

//

// If the IMMED bit is 1, status is returned as soon

// as the operation is initiated. If the IMMED bit

// is 0, status is not returned until the operation

// is completed.

//

#define CDB_RETURN_ON_COMPLETION   0

#define CDB_RETURN_IMMEDIATE       1

//

// Inquiry buffer structure. This is the data returned from the target

// after it receives an inquiry.

//

// This structure may be extended by the number of bytes specified

// in the field AdditionalLength. The defined size constant only

// includes fields through ProductRevisionLevel.

//

// The NT SCSI drivers are only interested in the first 36 bytes of data.

//

#define INQUIRYDATABUFFERSIZE 36

typedef struct _INQUIRYDATA {

UCHAR DeviceType : 5;

UCHAR DeviceTypeQualifier : 3;

UCHAR DeviceTypeModifier : 7;

UCHAR RemovableMedia : 1;

UCHAR Versions;

UCHAR ResponseDataFormat;

UCHAR AdditionalLength;

UCHAR Reserved[2];

UCHAR SoftReset : 1;

UCHAR CommandQueue : 1;

UCHAR Reserved2 : 1;

UCHAR LinkedCommands : 1;

UCHAR Synchronous : 1;

UCHAR Wide16Bit : 1;

UCHAR Wide32Bit : 1;

UCHAR RelativeAddressing : 1;

UCHAR VendorId[8];

UCHAR ProductId[16];

UCHAR ProductRevisionLevel[4];

UCHAR VendorSpecific[20];

UCHAR Reserved3[40];

} INQUIRYDATA, *PINQUIRYDATA;

//

// Inquiry defines. Used to interpret data returned from target as result

// of inquiry command.

//

// DeviceType field

//

#define DIRECT_ACCESS_DEVICE            0x00    // disks

#define SEQUENTIAL_ACCESS_DEVICE        0x01    // tapes

#define PRINTER_DEVICE                  0x02    // printers

#define PROCESSOR_DEVICE                0x03    // scanners, printers, etc

#define WRITE_ONCE_READ_MULTIPLE_DEVICE 0x04    // worms

#define READ_ONLY_DIRECT_ACCESS_DEVICE  0x05    // cdroms

#define SCANNER_DEVICE                  0x06    // scanners

#define OPTICAL_DEVICE                  0x07    // optical disks

#define MEDIUM_CHANGER                  0x08    // jukebox

#define COMMUNICATION_DEVICE            0x09    // network

#define LOGICAL_UNIT_NOT_PRESENT_DEVICE 0x7F

#define DEVICE_QUALIFIER_NOT_SUPPORTED  0x03

//

// DeviceTypeQualifier field

//

#define DEVICE_CONNECTED 0x00

//

// Sense Data Format

//

typedef struct _SENSE_DATA {

UCHAR ErrorCode:7;

UCHAR Valid:1;

UCHAR SegmentNumber;

UCHAR SenseKey:4;

UCHAR Reserved:1;

UCHAR IncorrectLength:1;

UCHAR EndOfMedia:1;

UCHAR FileMark:1;

UCHAR Information[4];

UCHAR AdditionalSenseLength;

UCHAR CommandSpecificInformation[4];

UCHAR AdditionalSenseCode;

UCHAR AdditionalSenseCodeQualifier;

UCHAR FieldReplaceableUnitCode;

UCHAR SenseKeySpecific[3];

} SENSE_DATA, *PSENSE_DATA;

//

// Default request sense buffer size

//

#define SENSE_BUFFER_SIZE 18

//

// Sense codes

//

#define SCSI_SENSE_NO_SENSE         0x00

#define SCSI_SENSE_RECOVERED_ERROR  0x01

#define SCSI_SENSE_NOT_READY        0x02

#define SCSI_SENSE_MEDIUM_ERROR     0x03

#define SCSI_SENSE_HARDWARE_ERROR   0x04

#define SCSI_SENSE_ILLEGAL_REQUEST  0x05

#define SCSI_SENSE_UNIT_ATTENTION   0x06

#define SCSI_SENSE_DATA_PROTECT     0x07

#define SCSI_SENSE_BLANK_CHECK      0x08

#define SCSI_SENSE_UNIQUE           0x09

#define SCSI_SENSE_COPY_ABORTED     0x0A

#define SCSI_SENSE_ABORTED_COMMAND  0x0B

#define SCSI_SENSE_EQUAL            0x0C

#define SCSI_SENSE_VOL_OVERFLOW     0x0D

#define SCSI_SENSE_MISCOMPARE       0x0E

#define SCSI_SENSE_RESERVED         0x0F

//

// Additional tape bit

//

#define SCSI_ILLEGAL_LENGTH         0x20

#define SCSI_EOM                    0x40

#define SCSI_FILE_MARK              0x80

//

// Additional Sense codes

//

#define SCSI_ADSENSE_NO_SENSE       0x00

#define SCSI_ADSENSE_LUN_NOT_READY  0x04

#define SCSI_ADSENSE_ILLEGAL_COMMAND 0x20

#define SCSI_ADSENSE_ILLEGAL_BLOCK  0x21

#define SCSI_ADSENSE_INVALID_LUN    0x25

#define SCSI_ADSENSE_INVALID_CDB    0x24

#define SCSI_ADSENSE_MUSIC_AREA     0xA0

#define SCSI_ADSENSE_DATA_AREA      0xA1

#define SCSI_ADSENSE_VOLUME_OVERFLOW 0xA7

#define SCSI_ADSENSE_NO_MEDIA_IN_DEVICE 0x3a

#define SCSI_ADWRITE_PROTECT        0x27

#define SCSI_ADSENSE_MEDIUM_CHANGED 0x28

#define SCSI_ADSENSE_BUS_RESET      0x29

#define SCSI_ADSENSE_TRACK_ERROR    0x14

#define SCSI_ADSENSE_SEEK_ERROR     0x15

#define SCSI_ADSENSE_REC_DATA_NOECC 0x17

#define SCSI_ADSENSE_REC_DATA_ECC   0x18

//

// Additional sense code qualifier

//

#define SCSI_SENSEQ_FORMAT_IN_PROGRESS 0x04

#define SCSI_SENSEQ_INIT_COMMAND_REQUIRED 0x02

#define SCSI_SENSEQ_MANUAL_INTERVENTION_REQUIRED 0x03

#define SCSI_SENSEQ_BECOMING_READY 0x01

#define SCSI_SENSEQ_FILEMARK_DETECTED 0x01

#define SCSI_SENSEQ_SETMARK_DETECTED 0x03

#define SCSI_SENSEQ_END_OF_MEDIA_DETECTED 0x02

#define SCSI_SENSEQ_BEGINNING_OF_MEDIA_DETECTED 0x04

//

// Read Capacity Data - returned in Big Endian format

//

typedef struct _READ_CAPACITY_DATA {

ULONG LogicalBlockAddress;

ULONG BytesPerBlock;

} READ_CAPACITY_DATA, *PREAD_CAPACITY_DATA;

//

// Read Block Limits Data - returned in Big Endian format

// This structure returns the maximum and minimum block

// size for a TAPE device.

//

typedef struct _READ_BLOCK_LIMITS {

UCHAR Reserved;

UCHAR BlockMaximumSize[3];

UCHAR BlockMinimumSize[2];

} READ_BLOCK_LIMITS_DATA, *PREAD_BLOCK_LIMITS_DATA;

//

// Mode data structures.

//

//

// Define Mode parameter header.

//

typedef struct _MODE_PARAMETER_HEADER {

UCHAR ModeDataLength;

UCHAR MediumType;

UCHAR DeviceSpecificParameter;

UCHAR BlockDescriptorLength;

}MODE_PARAMETER_HEADER, *PMODE_PARAMETER_HEADER;

#define MODE_FD_SINGLE_SIDE     0x01

#define MODE_FD_DOUBLE_SIDE     0x02

#define MODE_FD_MAXIMUM_TYPE    0x1E

#define MODE_DSP_FUA_SUPPORTED  0x10

#define MODE_DSP_WRITE_PROTECT  0x80

//

// Define the mode parameter block.

//

typedef struct _MODE_PARAMETER_BLOCK {

UCHAR DensityCode;

UCHAR NumberOfBlocks[3];

UCHAR Reserved;

UCHAR BlockLength[3];

}MODE_PARAMETER_BLOCK, *PMODE_PARAMETER_BLOCK;

//

// Define Disconnect-Reconnect page.

//

typedef struct _MODE_DISCONNECT_PAGE {

UCHAR PageCode : 6;

UCHAR Reserved : 1;

UCHAR PageSavable : 1;

UCHAR PageLength;

UCHAR BufferFullRatio;

UCHAR BufferEmptyRatio;

UCHAR BusInactivityLimit[2];

UCHAR BusDisconnectTime[2];

UCHAR BusConnectTime[2];

UCHAR MaximumBurstSize[2];

UCHAR DataTransferDisconnect : 2;

UCHAR Reserved2[3];

}MODE_DISCONNECT_PAGE, *PMODE_DISCONNECT_PAGE;

//

// Define mode caching page.

//

typedef struct _MODE_CACHING_PAGE {

UCHAR PageCode : 6;

UCHAR Reserved : 1;

UCHAR PageSavable : 1;

UCHAR PageLength;

UCHAR ReadDisableCache : 1;

UCHAR MultiplicationFactor : 1;

UCHAR WriteCacheEnable : 1;

UCHAR Reserved2 : 5;

UCHAR WriteRetensionPriority : 4;

UCHAR ReadRetensionPriority : 4;

UCHAR DisablePrefetchTransfer[2];

UCHAR MinimumPrefetch[2];

UCHAR MaximumPrefetch[2];

UCHAR MaximumPrefetchCeiling[2];

}MODE_CACHING_PAGE, *PMODE_CACHING_PAGE;

//

// Define mode flexible disk page.

//

typedef struct _MODE_FLEXIBLE_DISK_PAGE {

UCHAR PageCode : 6;

UCHAR Reserved : 1;

UCHAR PageSavable : 1;

UCHAR PageLength;

UCHAR TransferRate[2];

UCHAR NumberOfHeads;

UCHAR SectorsPerTrack;

UCHAR BytesPerSector[2];

UCHAR NumberOfCylinders[2];

UCHAR StartWritePrecom[2];

UCHAR StartReducedCurrent[2];

UCHAR StepRate[2];

UCHAR StepPluseWidth;

UCHAR HeadSettleDelay[2];

UCHAR MotorOnDelay;

UCHAR MotorOffDelay;

UCHAR Reserved2 : 5;

UCHAR MotorOnAsserted : 1;

UCHAR StartSectorNumber : 1;

UCHAR TrueReadySignal : 1;

UCHAR StepPlusePerCyclynder : 4;

UCHAR Reserved3 : 4;

UCHAR WriteCompenstation;

UCHAR HeadLoadDelay;

UCHAR HeadUnloadDelay;

UCHAR Pin2Usage : 4;

UCHAR Pin34Usage : 4;

UCHAR Pin1Usage : 4;

UCHAR Pin4Usage : 4;

UCHAR MediumRotationRate[2];

UCHAR Reserved4[2];

}MODE_FLEXIBLE_DISK_PAGE, *PMODE_FLEXIBLE_DISK_PAGE;

//

// Define mode format page.

//

typedef struct _MODE_FORMAT_PAGE {

UCHAR PageCode : 6;

UCHAR Reserved : 1;

UCHAR PageSavable : 1;

UCHAR PageLength;

UCHAR TracksPerZone[2];

UCHAR AlternateSectorsPerZone[2];

UCHAR AlternateTracksPerZone[2];

UCHAR AlternateTracksPerLogicalUnit[2];

UCHAR SectorsPerTrack[2];

UCHAR BytesPerPhysicalSector[2];

UCHAR Interleave[2];

UCHAR TrackSkewFactor[2];

UCHAR CylinderSkewFactor[2];

UCHAR Reserved2 : 4;

UCHAR SurfaceFirst : 1;

UCHAR RemovableMedia : 1;

UCHAR HardSectorFormating : 1;

UCHAR SoftSectorFormating : 1;

UCHAR Reserved3[2];

}MODE_FORMAT_PAGE, *PMODE_FORMAT_PAGE;

//

// Define rigid disk driver geometry page.

//

typedef struct _MODE_RIGID_GEOMETRY_PAGE {

UCHAR PageCode : 6;

UCHAR Reserved : 1;

UCHAR PageSavable : 1;

UCHAR PageLength;

UCHAR NumberOfCylinders[2];

UCHAR NumberOfHeads;

UCHAR StartWritePrecom[2];

UCHAR StartReducedCurrent[2];

UCHAR DriveStepRate[2];

UCHAR LandZoneCyclinder[2];

UCHAR RotationalPositionLock : 2;

UCHAR Reserved2 : 6;

UCHAR RotationOffset;

UCHAR Reserved3;

UCHAR RoataionRate[2];

UCHAR Reserved4[2];

}MODE_RIGID_GEOMETRY_PAGE, *PMODE_RIGID_GEOMETRY_PAGE;

//

// Define read write recovery page

//

typedef struct _MODE_READ_WRITE_RECOVERY_PAGE {

UCHAR PageCode : 6;

UCHAR Reserved1 : 1;

UCHAR PSBit : 1;

UCHAR PageLength;

UCHAR DCRBit : 1;

UCHAR DTEBit : 1;

UCHAR PERBit : 1;

UCHAR EERBit : 1;

UCHAR RCBit : 1;

UCHAR TBBit : 1;

UCHAR ARRE : 1;

UCHAR AWRE : 1;

UCHAR ReadRetryCount;

UCHAR Reserved4[4];

UCHAR WriteRetryCount;

UCHAR Reserved5[3];

} MODE_READ_WRITE_RECOVERY_PAGE, *PMODE_READ_WRITE_RECOVERY_PAGE;

//

// Mode parameter list block descriptor -

// set the block length for reading/writing

//

//

#define MODE_BLOCK_DESC_LENGTH               8

typedef struct _MODE_PARM_READ_WRITE {

MODE_PARAMETER_HEADER  ParameterListHeader;  // List Header Format

MODE_PARAMETER_BLOCK   ParameterListBlock;   // List Block Descriptor

} MODE_PARM_READ_WRITE_DATA, *PMODE_PARM_READ_WRITE_DATA;

//

// Tape definitions

//

typedef struct _TAPE_POSITION_DATA {

UCHAR Reserved1:2;

UCHAR BlockPositionUnsupported:1;

UCHAR Reserved2:3;

UCHAR EndOfPartition:1;

UCHAR BeginningOfPartition:1;

UCHAR PartitionNumber;

USHORT Reserved3;

UCHAR FirstBlock[4];

UCHAR LastBlock[4];

UCHAR Reserved4;

UCHAR NumberOfBlocks[3];

UCHAR NumberOfBytes[4];

} TAPE_POSITION_DATA, *PTAPE_POSITION_DATA;

//

// This structure is used to convert little endian

// ULONGs to SCSI CDB 4 byte big endians values.

//

typedef struct _FOUR_BYTE {

UCHAR Byte0;

UCHAR Byte1;

UCHAR Byte2;

UCHAR Byte3;

} FOUR_BYTE, *PFOUR_BYTE;

//

// Byte reversing macro for converting

// between big- and little-endian formats

//

#define REVERSE_BYTES(Destination, Source) {                \

PFOUR_BYTE d = (PFOUR_BYTE)(Destination);               \

PFOUR_BYTE s = (PFOUR_BYTE)(Source);                    \

d->Byte3 = s->Byte0;                                    \

d->Byte2 = s->Byte1;                                    \

d->Byte1 = s->Byte2;                                    \

d->Byte0 = s->Byte3;                                    \

}

//

// This macro has the effect of Bit = log2(Data)

//

#define WHICH_BIT(Data, Bit) {                      \

for (Bit = 0; Bit < 32; Bit++) {                \

if ((Data >> Bit) == 1) {                   \

break;                                  \

}                                           \

}                                               \

}

#if DBG

#define DebugPrint(x) ScsiDebugPrint x

#else

#define DebugPrint(x)

#endif // DBG

//

// Define Device Configuration Page

//

typedef struct _MODE_DEVICE_CONFIGURATION_PAGE {

UCHAR PageCode : 6;

UCHAR Reserved1 : 1;

UCHAR PS : 1;

UCHAR PageLength;

UCHAR ActiveFormat : 5;

UCHAR CAFBit : 1;

UCHAR CAPBit : 1;

UCHAR Reserved2 : 1;

UCHAR ActivePartition;

UCHAR WriteBufferFullRatio;

UCHAR ReadBufferEmptyRatio;

UCHAR WriteDelayTime[2];

UCHAR REW : 1;

UCHAR RBO : 1;

UCHAR SOCF : 2;

UCHAR AVC : 1;

UCHAR RSmk : 1;

UCHAR BIS : 1;

UCHAR DBR : 1;

UCHAR GapSize;

UCHAR Reserved3 : 3;

UCHAR SEW : 1;

UCHAR EEG : 1;

UCHAR EODdefined : 3;

UCHAR BufferSize[3];

UCHAR DCAlgorithm;

UCHAR Reserved4;

} MODE_DEVICE_CONFIGURATION_PAGE, *PMODE_DEVICE_CONFIGURATION_PAGE;

//

// Define Medium Partition Page

//

typedef struct _MODE_MEDIUM_PARTITION_PAGE {

UCHAR PageCode : 6;

UCHAR Reserved1 : 1;

UCHAR PSBit : 1;

UCHAR PageLength;

UCHAR MaximumAdditionalPartitions;

UCHAR AdditionalPartitionDefined;

UCHAR Reserved2 : 3;

UCHAR PSUMBit : 2;

UCHAR IDPBit : 1;

UCHAR SDPBit : 1;

UCHAR FDPBit : 1;

UCHAR MediumFormatRecognition;

UCHAR Reserved3[2];

UCHAR Partition0Size[2];

UCHAR Partition1Size[2];

} MODE_MEDIUM_PARTITION_PAGE, *PMODE_MEDIUM_PARTITION_PAGE;

//

// Define Data Compression Page

//

typedef struct _MODE_DATA_COMPRESSION_PAGE {

UCHAR PageCode : 6;

UCHAR Reserved1 : 2;

UCHAR PageLength;

UCHAR Reserved2 : 6;

UCHAR DCC : 1;

UCHAR DCE : 1;

UCHAR Reserved3 : 5;

UCHAR RED : 2;

UCHAR DDE : 1;

UCHAR CompressionAlgorithm[4];

UCHAR DecompressionAlgorithm[4];

UCHAR Reserved4[4];

} MODE_DATA_COMPRESSION_PAGE, *PMODE_DATA_COMPRESSION_PAGE;

//

// Define capabilites and mechanical status page.

//

typedef struct _MODE_CAPABILITIES_PAGE {

UCHAR PageCode : 6;

UCHAR Reserved1 : 1;

UCHAR PageSavable : 1;

UCHAR PageLength;

UCHAR Reserved2[2];

UCHAR RO : 1;

UCHAR Reserved3 : 1;

UCHAR LSEL : 1;

UCHAR LSEN : 1;

UCHAR Reserved4 : 1;

UCHAR SPREV : 1;

UCHAR Reserved5 : 1;

UCHAR WFM : 1;

UCHAR FBC : 1;

UCHAR LIMDL : 1;

UCHAR FAPP : 1;

UCHAR EFMT : 1;

UCHAR Reserved6 : 1;

UCHAR QFA : 1;

UCHAR PHYBLK : 1;

UCHAR LOGBLK : 1;

UCHAR LOCK : 1;

UCHAR LOCKED : 1;

UCHAR PREVENT : 1;

UCHAR UNLOAD : 1;

UCHAR Reserved7 : 2;

UCHAR ECC : 1;

UCHAR CMPRS : 1;

UCHAR Reserved8 : 1;

UCHAR BLK512 : 1;

UCHAR BLK1024 : 1;

UCHAR Reserved9 : 4;

UCHAR SLOWB : 1;

UCHAR MaximumSpeedSupported[2];

UCHAR MaximumStoredDefectedListEntries[2];

UCHAR ContinuousTransferLimit[2];

UCHAR CurrentSpeedSelected[2];

UCHAR BufferSize[2];

UCHAR Reserved10[4];

} MODE_CAPABILITIES_PAGE, *PMODE_CAPABILITIES_PAGE;

typedef struct _MODE_CAP_PAGE {

MODE_PARAMETER_HEADER   ParameterListHeader;

MODE_PARAMETER_BLOCK    ParameterListBlock;

MODE_CAPABILITIES_PAGE  CapabilitiesPage;

} MODE_CAP_PAGE, *PMODE_CAP_PAGE;

//

// Mode parameter list header and medium partition page -

// used in creating partitions

//

typedef struct _MODE_MEDIUM_PART_PAGE {

MODE_PARAMETER_HEADER       ParameterListHeader;

MODE_MEDIUM_PARTITION_PAGE  MediumPartPage;

} MODE_MEDIUM_PART_PAGE, *PMODE_MEDIUM_PART_PAGE;

typedef struct _MODE_MEDIUM_PART_PAGE_PLUS {

MODE_PARAMETER_HEADER       ParameterListHeader;

MODE_PARAMETER_BLOCK        ParameterListBlock;

MODE_MEDIUM_PARTITION_PAGE  MediumPartPage;

} MODE_MEDIUM_PART_PAGE_PLUS, *PMODE_MEDIUM_PART_PAGE_PLUS;

//

// Mode parameters for retrieving tape or media information

//

typedef struct _MODE_TAPE_MEDIA_INFORMATION {

MODE_PARAMETER_HEADER       ParameterListHeader;

MODE_PARAMETER_BLOCK        ParameterListBlock;

MODE_MEDIUM_PARTITION_PAGE  MediumPartPage;

} MODE_TAPE_MEDIA_INFORMATION, *PMODE_TAPE_MEDIA_INFORMATION;

//

// Mode parameter list header and device configuration page -

// used in retrieving device configuration information

//

typedef struct _MODE_DEVICE_CONFIG_PAGE {

MODE_PARAMETER_HEADER       ParameterListHeader;

MODE_DEVICE_CONFIGURATION_PAGE  DeviceConfigPage;

} MODE_DEVICE_CONFIG_PAGE, *PMODE_DEVICE_CONFIG_PAGE;

//

// Mode parameter list header and data compression page -

// used in retrieving data compression information

//

typedef struct _MODE_DATA_COMPRESS_PAGE {

MODE_PARAMETER_HEADER       ParameterListHeader;

MODE_DATA_COMPRESSION_PAGE  DataCompressPage;

} MODE_DATA_COMPRESS_PAGE, *PMODE_DATA_COMPRESS_PAGE;

//

// Tape/Minitape definition.

//

typedef

BOOLEAN

(*TAPE_VERIFY_INQUIRY_ROUTINE)(

IN  PINQUIRYDATA            InquiryData,

IN  PMODE_CAPABILITIES_PAGE ModeCapabilitiesPage

);

typedef

VOID

(*EXTENSION_INIT_ROUTINE)(

IN  PVOID                   MinitapeExtension,

IN  PINQUIRYDATA            InquiryData,

IN  PMODE_CAPABILITIES_PAGE ModeCapabilitiesPage

);

typedef enum _TAPE_STATUS {

TAPE_STATUS_SEND_SRB_AND_CALLBACK,

TAPE_STATUS_SUCCESS,

TAPE_STATUS_INSUFFICIENT_RESOURCES,

TAPE_STATUS_NOT_IMPLEMENTED,

TAPE_STATUS_INVALID_DEVICE_REQUEST,

TAPE_STATUS_INVALID_PARAMETER,

TAPE_STATUS_MEDIA_CHANGED,

TAPE_STATUS_BUS_RESET,

TAPE_STATUS_SETMARK_DETECTED,

TAPE_STATUS_FILEMARK_DETECTED,

TAPE_STATUS_BEGINNING_OF_MEDIA,

TAPE_STATUS_END_OF_MEDIA,

TAPE_STATUS_BUFFER_OVERFLOW,

TAPE_STATUS_NO_DATA_DETECTED,

TAPE_STATUS_EOM_OVERFLOW,

TAPE_STATUS_NO_MEDIA,

TAPE_STATUS_IO_DEVICE_ERROR,

TAPE_STATUS_UNRECOGNIZED_MEDIA,

TAPE_STATUS_DEVICE_NOT_READY,

TAPE_STATUS_MEDIA_WRITE_PROTECTED,

TAPE_STATUS_DEVICE_DATA_ERROR,

TAPE_STATUS_NO_SUCH_DEVICE,

TAPE_STATUS_INVALID_BLOCK_LENGTH,

TAPE_STATUS_IO_TIMEOUT,

TAPE_STATUS_DEVICE_NOT_CONNECTED,

TAPE_STATUS_DATA_OVERRUN,

TAPE_STATUS_DEVICE_BUSY

} TAPE_STATUS, *PTAPE_STATUS;

typedef

VOID

(*TAPE_ERROR_ROUTINE)(

IN      PVOID               MinitapeExtension,

IN      PSCSI_REQUEST_BLOCK Srb,

IN OUT  PTAPE_STATUS        TapeStatus

);

#define TAPE_RETRY_MASK 0x0000FFFF

#define IGNORE_ERRORS   0x00010000

typedef

TAPE_STATUS

(*TAPE_PROCESS_COMMAND_ROUTINE)(

IN OUT  PVOID               MinitapeExtension,

IN OUT  PVOID               CommandExtension,

IN OUT  PVOID               CommandParameters,

IN OUT  PSCSI_REQUEST_BLOCK Srb,

IN      ULONG               CallNumber,

IN OUT  PULONG              RetryFlags

);

typedef struct _TAPE_INIT_DATA {

TAPE_VERIFY_INQUIRY_ROUTINE     VerifyInquiry;

ULONG                           MinitapeExtensionSize;

EXTENSION_INIT_ROUTINE          ExtensionInit;          /* OPTIONAL */

ULONG                           DefaultTimeOutValue;    /* OPTIONAL */

TAPE_ERROR_ROUTINE              TapeError;              /* OPTIONAL */

ULONG                           CommandExtensionSize;

TAPE_PROCESS_COMMAND_ROUTINE    CreatePartition;

TAPE_PROCESS_COMMAND_ROUTINE    Erase;

TAPE_PROCESS_COMMAND_ROUTINE    GetDriveParameters;

TAPE_PROCESS_COMMAND_ROUTINE    GetMediaParameters;

TAPE_PROCESS_COMMAND_ROUTINE    GetPosition;

TAPE_PROCESS_COMMAND_ROUTINE    GetStatus;

TAPE_PROCESS_COMMAND_ROUTINE    Prepare;

TAPE_PROCESS_COMMAND_ROUTINE    SetDriveParameters;

TAPE_PROCESS_COMMAND_ROUTINE    SetMediaParameters;

TAPE_PROCESS_COMMAND_ROUTINE    SetPosition;

TAPE_PROCESS_COMMAND_ROUTINE    WriteMarks;

} TAPE_INIT_DATA, *PTAPE_INIT_DATA;

SCSIPORT_API

ULONG

ScsiTapeInitialize(

IN  PVOID           Argument1,

IN  PVOID           Argument2,

IN  PTAPE_INIT_DATA TapeInitData

);

SCSIPORT_API

BOOLEAN

ScsiTapeAllocateSrbBuffer(

IN OUT  PSCSI_REQUEST_BLOCK Srb,

IN      ULONG               SrbBufferSize

);

SCSIPORT_API

VOID

ScsiTapeZeroMemory(

IN OUT  PVOID   Buffer,

IN      ULONG   BufferSize

);

SCSIPORT_API

ULONG

ScsiTapeCompareMemory(

IN OUT  PVOID   Source1,

IN OUT  PVOID   Source2,

IN      ULONG   Length

);

//

// defines for QIC tape density codes

//

#define QIC_XX   0   // ????

#define QIC_24   5   // 0x05

#define QIC_120  15  // 0x0F

#define QIC_150  16  // 0x10

#define QIC_525  17  // 0x11

#define QIC_1350 18  // 0x12

#define QIC_1000 21  // 0x15

#define QIC_2100 31  // 0x1F

#define QIC_2GB  34  // 0x22

//

// defines for QIC tape media codes

//

#define DCXXXX   0

#define DC300    1

#define DC300XLP 2

#define DC615    3

#define DC600    4

#define DC6037   5

#define DC6150   6

#define DC6250   7

#define DC6320   8

#define DC6525   9

#define DC9135SL 33  //0x21

#define DC9210   34  //0x22

#define DC9135   35  //0x23

#define DC9100   36  //0x24

#define DC9120   37  //0x25

#define DC9100SL 38  //0x26

#define DC9164   39  //0x27

#define DC9210XL 50  //0x32

#define DC10GB   51  //0x33

#define DC9100XL 52  //0x34

#define DC9120XL 53  //0x35

#define DC9164XL 55  //0x37

//

// defines for translation reference point

//

#define NOT_FROM_BOT 0

#define FROM_BOT 1

//

// info/structure returned by/from

// TapeLogicalBlockToPhysicalBlock( )

//

typedef struct _TAPE_PHYS_POSITION {

ULONG SeekBlockAddress;

ULONG SpaceBlockCount;

} TAPE_PHYS_POSITION, PTAPE_PHYS_POSITION;

//

// function prototypes

//

TAPE_PHYS_POSITION

TapeLogicalBlockToPhysicalBlock(

IN UCHAR DensityCode,

IN ULONG LogicalBlockAddress,

IN ULONG BlockLength,

IN BOOLEAN FromBOT

);

ULONG

TapePhysicalBlockToLogicalBlock(

IN UCHAR DensityCode,

IN ULONG PhysicalBlockAddress,

IN ULONG BlockLength,

IN BOOLEAN FromBOT

);

#endif /* _MINITAPE_ */