LAN Manager - NetWare Connectivity

for

ARCNet Media

Author:	Len Smale (lens)

Date:		07/28/26

Revision:	Working Document

µ

1.	Purpose of This Document	3

2.	Requirements	3

3.	Non Requirements	4

4.	Current ARCNet Transports and Hardware	4

4.1.	ARCNet Manufacturers	4

4.1.1.	Standard Microsystems Corp. (SMC)	4

4.1.2.	Thomas Conrad (T-C)	5

4.2.	Protocol Interoperability between Manufacturers	5

4.2.1.	NetWare	5

4.2.2.	 NDIS LAN Manager/Encapsulated Ethernet	5

4.2.3.	TCP/IP	5

4.3.	ARCNet Protocol Formats	6

4.3.1.	Note on ARCNet Packet Representations	6

4.3.2.	ARCNet Media Header	6

4.3.3.	Novell's Packet Fragmentation Scheme	7

4.3.4.	NetWare Protocols	7

4.3.5.	Encapsulated Ethernet Protocol	7

4.3.6.	TCP/IP System Codes (RFC 1051)	8

5.	Functional Description of Solution	9

6.	Architecture Alternatives	10

6.1.	 ARCNet Packets Presented to Transport	10

6.2.	Ethernet Packets Presented to (IPX) Transport	10

6.2.1.	Create a "pseudo" NDIS driver	11

6.2.2.	Tightly couple to the ARCNet NDIS MAC driver.	11

7.	Architecture Selection	11

8.	High Level Software Design	12

8.1.	 New ARCFRAG TSR	12

8.2.	Enhancements to ARCNet MAC Driver	12

8.3.	PROTOCOL.INI Layout	13

8.4.	Installation Procedure	14

8.5.	Installation Changes	15

9.	Conformance Testing	16

10.	References	16

# 1	Purpose of This Document

NetWare Connectivity for LAN Manager 2.1 provides an object module, NWNDIS.OBJ,  that can be linked with IPX.OBJ to provide an executable image, IPX.EXE, which is an NDIS 2.0.1 compliant IPX transport for MS-DOS. This transport can then be used with LAN Manager transports (NETBEUI, XNS, etc.) to share the same NDIS MAC driver,

allowing both LAN Manager and NetWare networks to share the same network card for both protocol stacks.

The NWNDIS.OBJ module currently supports the Token Ring, and Ethernet (both DIX and IEEE 802.2/IEEE 802.3) MAC media header formats.

It does not, however,  support the ARCNet media header format.

Consequently, the current NetWare Connectivity does not allow LAN Manager and NetWare to share the same ARCNet network card.

The purpose of this document is to identify and outline the changes that should be made to the ARCNet NDIS MAC drivers and the NWNDIS.OBJ module so that LAN Manager and NetWare protocol stacks can share the same network card as they can for other media types.

# 2	Requirements

The main requirement is that the solution should support existing NetWare packet formats, so that clients can communicate with existing NetWare servers without changing the server communication card drivers.

Clearly,  LAN Manager protocols (at least NETBEUI) should continue to work over the ARCNet NDIS driver. Existing LAN Manager ARCNet transport drivers should not have to be modified to support a NetWare connectivity environment. (This requirement means that the current LAN Manager Ethernet Encapsulation protocol on ARCNet must be supported).

The packaging should be simple and easy to install, with a straightforward PROTOCOL.INI - ideally it should look like any other NDIS driver.

# 3	Non Requirements

There are  several non-requirements:

·	Preserve the existing Ethernet Encapsulation format. This would be nice, but no one has claimed this as a requirement.

·	Support other protocols (such as Appletalk) from the one ARCNet communications card. NOTE: This is significant because it says that connectivity between LAN Manager servers with Appletalk networks running over ARCNet into the servers is not being addressed by this document.

·	Preserve the high performance benefit of  the NDIS interface over the ODLI interface for the NetWare protocol.

# 4	Current ARCNet Transports and Hardware

## 5	ARCNet Manufacturers

There are currently two major manufacturers of  ARCNet network cards:

·	Thomas Conrad (T-C)

·	Standard Microsystems Corp. (SMC)

### 6	Standard Microsystems Corp. (SMC)

The SMC cards are the same product line that is supported by Novell - the PC130,  PC220, and PS110 boards are major versions that are documented in "Novell Standard Microsystems ARCNET Supplement".

When used with NetWare, the IPX.COM driver generates packets in the standard NetWare form for ARCNet as described below in the section "NetWare System Code". The NetWare ARCNet driver is shipped with NetWare as the RX-Net driver.

When used with LAN Manager, the NDIS ARCNet MAC driver claims to be an Ethernet DIX+802.3 MAC in the MAC Service Specific Characteristics Table, and encapsulates a transport's request to send Ethernet packets in  ARCNet packets as described below in the section "Encapsulated Ethernet System Code".

The SMC NDIS ARCNet driver was written by DWB Associates and is now maintained directly by Microsoft.

### 7	Thomas Conrad (T-C)

As do SMC, the Thomas Conrad ARCNet network cards support both NetWare

in the Novell defined standard form and LAN Manager using encapsulated Ethernet.

The ARCNet MAC drivers were written by T-C and the source is maintained by them.

The T-C NetWare driver modules are not shipped with the NetWare driver diskettes, but are available from T-C via a bulletin board. I am in the process of obtaining these drivers.

## 8	Protocol Interoperability between Manufacturers

### 9	NetWare

Whilst I have not verified it yet, Jason Spofford of T-C says that the T-C cards and the SMC cards interoperate with NetWare protocols. This seems reasonable since the NetWare packet format is defined by Novell (see section  "NetWare System Code" below for more information).

### 10	 NDIS LAN Manager/Encapsulated Ethernet

Both the SMC and the T-C NDIS drivers use encapsulated Ethernet for support of LAN Manager protocols. However, they do not inter-operate. They both use the same ARCNet protocol ID (E8 hex),  and the same scheme for converting Ethernet addresses to ARCNet adresses. The ARCNet and Ethernet byte counts differ by one  byte which consists of an Ethrnet pad byte. This deserves further investigation, but is not really part of NetWare connectivity.

### 11	TCP/IP

There are several forms of TCP/IP that can exist on ARCNet, including:

·	NetWare TCP/IP, where IP packets are encapsulated inside IPX packets.

·	LAN Manager TCP/IP, where IP packets are in the encasulated Ethernet packets.

·	Native TCP/IP on ARCNet, where the IP (and  ARP) packets use their own ARCNet protocol IDs, as defined in RFC 1051.

These various forms of TCP/IP do not interoperate since they use different formats at the media level (i.e. different ARCNet protocol types).

## 12	ARCNet Protocol Formats

### 13	Note on ARCNet Packet Representations

There are  various different ARCNet packet representations. This can be somewhat confusing, so the differences are listed below:

·	RFC 1051 describes the packets as they are represented on the media, with:   ALERT, SOH, and CRC  fields included; duplicated destination address byte; and the COUNT being the number of bytes of data including the ARCNet system code.

·	Novell's ARCNET Packet Header Definition Standard describes the packets as they are stored and received by the SMC card's hardware RAM buffer, with the data offset field being the offset of  the protocol ID from the end of the buffer.

Novell's data offset field is the same information as the COUNT field, but with a different binary representation.

·	NetWork General's Sniffer shows the packets as if they are in the long packet format (i.e. packets of size 257-508 bytes, excluding ARCNet MAC header, but including protocol ID field), even if the packets are short packets (i.e. packets of size 1-253 bytes). It does this by changing the COUNT field from short packet format to long packet format. This is persumably a hack for the Sniffer's decoder software to correctly decompose packets into fields. Also the NetWare decoder does not correctly interpret the Split Flag field - thus read/write NCP packets are not decoded correctly.

·	The T-C fax describes the packets in a the  format presented on the media.

It incorrectly says that the DIX Ethernet II frame size reported by the Encapsulated Ethernet MAC driver is 508 bytes (it is 507 bytes), overlooking the system code/protocol ID.

### 14	ARCNet Media Header

ARCNet packets have a standard media header format:

| Source Address | Byte |
| --- | --- |
| Destination Address | Byte |
| Data Length | 1-2 Bytes |
| Protocol Type | Byte |

This header is immediately followed by the data when given to a transport driver.

The data length field is 1-2 bytes depending upon the size of the data in the packet.

The data length is for the subsequent protocol type and all following packet data.

It is presented to the transport protocols as a simple byte count.

The protocol type is not mandatory for the ARCNet card drivers, but all protocols (that can inter-operate with other protocols) contain it.

### 15	Novell's Packet Fragmentation Scheme

The NetWare NCP protocol requires the IPX transport support as a minimum IPX packet size of  576 bytes. This enables reads/writes of 512 bytes of data with 64 bytes of NCP/IPX header information.

Because ARCNet has such a small maximum packet size (507 bytes of  data following the addressing information and protocol ID),  Novell defined a scheme whereby several data packets could be transmitted as several consecutive ARCNet packets and reconstructed at the reciever into one data packet. This is referred to as packet fragmentation.  This is done by placing fragmentation fields after the protocol ID field in the ARCNet data. The exact field formats are defined in the "Novell Standard Microsystems ARCNET Supplement" .

NOTE: Because this scheme is so useful, it is being adopted by other vendors.

In particular,  it is used by the Appletalk protocol on ARCNet which uses its own protocol ID. It thus has a wider scope than just NetWare protocols.

### 16	NetWare Protocols

NetWare IPX packets use a protocol ID of  FA. All of the other NetWare protocols (SPX, RIP, SAP, TCP/IP, etc. ) are layered upon IPX. The NetWare protocols obviously use the packet fragmentation scheme mentioned above.

### 17	Encapsulated Ethernet Protocol

LAN Manager uses the encapsulated Ethernet protocol (ID E8) mentioned above.

The ARCNet NDIS MAC driver presents itself as an Ethernet driver to the transport and actually transmits the Ethernet headers across the network. The driver constructs a fake Ethernet address by putting the ARCNet card's address in the lower byte of the Ethernet address. No fragmentation scheme is employed, limiting the maximum Ethernet packet size to 507 bytes, including the Ethernet header.

Ethernet multicasts are emulated by doing ARCNet broadcasts. Functional/group addresses are not supported.

Note: It would be an improvement to this scheme if the LAN Manager encapsulated Ethernet protocol employed the Novell ARCNet fragmentation scheme to support the normal Ethernet maximum packet size.

### 18	TCP/IP System Codes (RFC 1051)

The RFC standard for TCP/IP over ARCNet uses assigned protocol IDs for the IP and ARP datagram protocols. Note that it does not use the Novell encapsulation scheme. The point here is that, even if  it is known whether or not certain protocol IDs use that Novell fragmentation scheme, an unknown protocol ID may or may not use this scheme.

# 19	Functional Description of Solution

The following diagram outlines the new NetWare systems and their relationship to the existing LAN Manager subsystems. The diagram includes data flow relationships, but does not specify how the subsystems are implemented.

µ §

# 20	Architecture Alternatives

The major decision is how to represent the ARCNet packets to the NDIS version of the IPX transport. There are two main options:

1.	As raw ARCNet packets.

2.	As Ethernet packets (DIX or Novell's form of 802.2 /802.3 without SSAP/DSAP fields).

These options have the following trade-offs:

## 21	 ARCNet Packets Presented to Transport

Benefits:

The best performance implementation because it does not require additional buffer copies when IPX.COM is receiving a fragmented IPX packet.

The best memory utilization because it does not require additional buffer copies when IPX.COM is receiving a fragmented IPX packet - buffers allocated can be specified  in NET.CFG using the usual NetWare scheme.

Disadvantages:

Requires that the Novell fragmentation scheme be implemented in the IPX.COM TSR - will not work with LAN Manager 2.1 NDIS IPX.COM.

## 22	Ethernet Packets Presented to (IPX) Transport

Benefits:

Works with the existing NDIS IPX.COM transport module - there would not be two NDIS versions of  IPX.COM causing upgrade problems.

Disadvantages:

Requires additional buffer space, increasing the buffer requirements and memory utilization by several Kbytes

This approach has two sub-options to implement the Novell ARCNet packets fragmentation/defragmentation:

### 23	Create a "pseudo" NDIS driver

The additional NDIS driver presents itself as an ARCNet NDIS transport driver to the ARCNet MAC driver and an Ethernet NDIS MAC driver to to the IPX transport driver. The pseudo NDIS driver sits between the ARCNet NDIS driver and the NDIS transports, managing ARCNet fragmentation for NetWare packets.

Benefits:

Since the existing NDIS specification becomes the specification for interfacing to the ARCNet MAC driver, the interface to OEMs writing ARCNet MAC drivers is the most simple.

Disadvantages:

Creates a complex PROTOCOL.INI configuration akin to using the UBLOOP driver for XNS.

### 24	Tightly couple to the ARCNet NDIS MAC driver.

The fragmentation scheme would be implemented as either object modules linked to the ARCNet MAC driver or a TSR with a call interface to the ARCNet MAC driver.

Benefits:

Has a simpler PROTOCOL.INI specification than the first alternative.

Disadvantages:

Requires additional interfaces to be defined and communicated to OEMs.

# 25	Architecture Selection

The recommended architecture is to use an additional NDIS driver/TSR between the ARCNet MAC driver and the IPX.COM, presenting the packets as Ethernet DIX packets to IPX.COM. Note that this would work with existing NDIS IPX.COM modules, whether they were ECONFIGed for DIX type 8137 or not.

This alternative is chosen because it has the benefits of minimal OEM support and also reduces PSS support because it works with the existing NDIS versions of IPX.COM.

The disdadvantages are not significant, given that high performance for the IPX protocol is not a requirement, and the increased buffer utilization of the Ethernet transport interface is not high.

In order to minimize installation and configuration complexity, the ARCNet MAC drivers will use a fixed module name for the interface to the additional driver/TSR for fragmentation. This means that only one ARCNet card can be supported to IPX at a time. This is not a serious disadvantage since the current NDIS IPX.COM implementation only supports one MAC driver.

# 26	High Level Software Design

## 27	 New ARCFRAG TSR

The new ARCFRAG device driver/TSR is loaded as a TSR so that its buffers can be freed when IPX is unloaded.

When loaded it attempts to bind to a MAC ARCNETNWFRAG.

The driver will be responsible for defragmenting individual ARCNet packets received from the network. If packets are received out-of-sequence, then all other packets from the same source address will be discarded awaiting a retransmit. If  the drivers uses up all of its receive buffers, then new packets received will be discarded until there are available receive buffers. If a receive buffer is partially filled, then it must be completely filled with one second, otherwise its data will be discarded.

The driver will also be responsible for fragmenting and sequence numbering buffers transmitted.

DPA capabilities for the TSR are provided by the TSR noting when the last transport bound to it does an unbind. If a second copy of the ARCFRAG program is then run, it will cause the TSR copy to exit (providing the interrupt vectors hooked by the TSR can be cleanly restored).

This mechanism allows the TSR to be removed from memory after unloading IPX.

## 28	Enhancements to ARCNet MAC Driver

The device driver will be able to be loaded with a line such as:

"DEVICE=C:\LANMAN.DOS\DRIVERS\ARCNET\MSARC\MSARC.DOS

Depending upon the options chosen, the driver will offer two NDIS configuration tables to PROTMAN, one for Ethernet and one for Novell fragmented ARCNet packets. The registration for Novell fragmented ARCNet packets is with the name ARCNETNWFRAG.

If the device driver is loaded twice at startup, the second load will do a "silent exit".

This is because the current installer will put the device driver in CONFIG.SYS twice, and this allows the extra line to be benign.

## 29	PROTOCOL.INI Layout

PROTOCOL.INI  is modified to contain lines for the new NDIS driver as well as a second NDIS interface to the ARCNet MAC driver which now has two NDIS interfaces (one for encapsulated Ethernet and one for Novell fragmented ARCNet packets). A typical fragment of PROTOCOL.INI is shown below for the case when NETBEUI and IPX are transports and the Microsoft drivers for the SMC ARCNet card are used:

[PROTMAN]

DRIVERNAME = PROTMAN$

DYNAMIC = YES

PRIORITY = NETBEUI

[NETBEUI_XIF]

Drivername = netbeui$

SESSIONS = 6

NCBS = 12

BINDINGS = "MSARC_NIF"

LANABASE = 0

[IPX_XIF]

; IPX for ARCNet

Drivername = IPX$

LOAD = ARCFRAG[U],IPXMARK[U],IPX[U]

UNLOAD = IPXREL[C],ARCFRAG[C]

BINDINGS = "MSARCNW_NIF"

[MSARCNW_NIF]

; protocol.ini section for the Microsoft ARCNet Driver NetWare  TSR

DRIVERNAME = ARCFRAG$

MAXRECEIVES = 4

[MSARC_NIF]

; protocol.ini section for the Microsoft ARCNet Driver

MEMORYBASE = 0xd000

IOBASE = 0x300

INTERRUPT = 3

MAXTRANSMITS = 40

DRIVERNAME = MSARC$

The [MSARC_NIF] section is the current ARCNet MAC driver's section for Ethernet encapsulation (protocol ID E8), and has a section name and PROTOCOL.INI section as required by the OEM device driver.

The [MSARCNW_NIF] section is a new section to pass Novell fragmented packets between the new Novell fragmentation TSR and the ARCNet MAC driver .

The MAXRECEIVES field indicates the number of buffers that should be allocated for reconstructing fragmented packets being received (one buffer per defragmentation). Each buffer should be large enough to contain an Ethernet packet.

The LOAD line for IPX is modified to load the new ARCFRAG TSR as well as the usual IPX TSRs.

## 30	Installation Procedure

The user does the following steps to perform the ARCNet installation:

1.	Installs NetWare Connectivity and the ARCNet MAC driver from the current LAN Manager distribution.

2.	Goes back into LAN Manager setup.

3.	From the "Configuration" Menu selects "Network Drivers".

4.	Chooses "Add New Configuration" and selects "Other Drivers".

5.	Inserts the disk containing the ARCNet updates (described in the section "Installation Changes" below) and selects it.

6.	Selects "Microsoft ARCNet Driver Extensions for NetWare".

7.	Receives a message asking if the user wants to overwrite the existing driver and selects "Yes".

8.	Is asked to select a protocol and selects "Other Protocol".

9.	From the same disk as above selects "IPX/SPX for ARCNet".

10.	Exits the screens back up-to the main "Networks Drivers" screen.

11.	Selects the "Microsoft ARCNet Driver" and assigns it the protocol used by LAN Manager (e.g. NETBEUI).

12.	Selects the "Microsoft ARCNet Driver Extensions for Netware" and assigns it the protocol "IPX/SPX for ARCNet".

13.	Exits setup, saving changes.

## 31	Installation Changes

The following describes the installation changes for the Microsoft software for SMCs ARCNet card. It is layed out for an install from a new "OEM" disk. Other installations are similar.

The following files are included on the NetWare install disk:

DRIVERS\NIF\MSARCNW.NIF

DRIVERS\ETHERNET\MSARC\MSARC.DOS

DRIVERS\ARCNET\ARCFRAG\ARCFRAG.COM

DRIVERS\ARCNET\ARCFRAG\PROTOCOL.INI

DRIVERS\XIF\IPX.XIF

DRIVERS\PROTOCOL\IPX\PROTOCOL.INI

The MSARCNW.NIF file is as follows:

Model = Microsoft ARCNet Driver Extensions for NetWare

Path = ARCNET\ARCFRAG

DeviceDriver = @LANROOT\DRIVERS\ETHERNET\MSARC\MSARC.DOS

DeviceName = MSARCNW

Type = NDIS

Mediatype = ARCNET

The DRIVERS\ETHERNET\MSARC\MSARC.DOS is a the enhanced SMC driver, unlike the one on the current LAN Manager installation disks.

The DRIVERS\ARCNET\ARCFRAG\ARCFRAG.COM is the new ARCFRAG TSR.

The DRIVERS\ARCNET\ARCFRAG\PROTOCOL.INI fragment above is as follows:

; protocol.ini section for the Microsoft ARCNet Driver  NetWare TSR.

DRIVERNAME = ARCFRAG$

MAXRECEIVES = 4

The DRIVERS\XIF\IPX.XIF is a XIF file used to update the PROTOCOL.INI for IPX, but does not copy IPX.COM or other files. Note that this file does not mention the IPXNDIS.DOS device driver, so none is placed in CONFIG.SYS.

Model = IPX/SPX for ARCNet

Path = PROTOCOL\IPX

DeviceName = ipx$

Type = NDIS_NONB

Class = IPX

The DRIVERS\PROTOCOL\IPX\PROTOCOL.INI is a version of PROTOCOL.INI for IPX that has been modified to allow for loading and unloading the ARCFRAG TSR:

; IPX for ARCNet

Drivername = IPX$

LOAD = ARCFRAG[U],IPXMARK[U],IPX[U]

UNLOAD = IPXREL[C],ARCFRAG[C]

# 32	Conformance Testing

The updated ARCNet drivers and their new functionality needs to be added to the NDIS conformance test program.

# 33	References

3Com/Microsoft LAN Manager Network Driver Interface Specification Version 2.0.1.

Novell Standard Microsystems ARCNET Supplement (A supplement to the Novell LAN Drivers Developers Guide).

Novell Standard Microsystems ARCNET Supplement ( A supplement to the NetWare 386 Documentation, included in the Supplements Manual).

Thomas Conrad Proposal, 21st August 1991

A Standard for the Transmission of IP Datagrams and ARP Packets over ARCNET Networks (RFC 1051).